8051 CROSS ASSEMBLER

USER'S MANUAL

MetaLink Corporation

Chandler, Arizona

MetaLink Corporation

P.O. BOX 1329

Chandler, Arizona  85244-1329

(602) 926-0797

TELEX: 4998050 MTLNK

FAX:  (602) 926-1198

PURCHASE TERMS AND CONDITIONS

Since MetaLink Corporation does business and is located solely in

the State of Arizona, such orders or agreements and the rights of

the parties hereunder  shall be governed by the laws of the State

of Arizona.

PAYMENTS TERMS:

Unless otherwise agreed upon, the TOTAL UNPAID BALANCE is due and

payable 30 days following the invoice date.

LIMITED WARRANTY:  METALINK MAKES NO WARRANTIES OTHER THAN THOSE

CONTAINED HEREIN AND METALINK EXPRESSLY DISCLAIMS ANY AND ALL

IMPLIED WARRANTIES, INCLUDING ANY WARRANTY OF FITNESS FOR A

PARTICULAR PURPOSE OR OF MERCHANTABILITY.

The foregoing limited warranty shall not apply unless Buyer has

paid for in full the MetaLink products.  Updates to the MetaLink

Assembler User's Manual and MetaLink Assembler software are

available free to Registered Buyer upon request for a one (1) year

period from the invoice date.

NOTICE

MetaLink Corp. reserves the right to make improvements in the

software product described in this manual as well as the manual

itself at any time and without notice.

DISCLAIMER OF ALL WARRANTIES AND LIABILITY

METALINK CORP. MAKES NO WARRANTIES, EITHER EXPRESSED OR IMPLIED,

WITH RESPECT TO THIS MANUAL OR WITH RESPECT TO THE SOFTWARE

DESCRIBED IN THIS MANUAL, ITS QUALITY, PERFORMANCE, MERCHANTABILITY,

OR FITNESS FOR ANY PARTICULAR PURPOSE.  METALINK CORP. SOFTWARE

IS SOLD OR LICENSED "AS IS".  IN NO EVENT SHALL METALINK CORP. BE

LIABLE FOR INCIDENTAL OR CONSEQUENTIAL DAMAGES RESULTING FROM ANY

DEFECT IN THE SOFTWARE.

Copyright (c) 1984, 1985, 1986, 1987, 1988, 1989 by MetaLink Corp.

All rights are reserved.  This manual may not, in whole or part,

be copied, photocopied, reproduced, translated, or reduced to any

electronic medium or machine readable form without the prior

agreement and written permission of MetaLink Corp.

MS-DOS is a trademark of Microsoft,  Inc.

IBMR is a registered trademark of IBM Corp.

IntelR is a registered trademark of Intel Corp.

MetaLink is a trademark of MetaLink Corp.

T A B L E   O F   C O N T E N T S

1. 8051 OVERVIEW............................................1-1

1.1. Introduction........................................1-1

1.2. 8051 Architecture...................................1-2

1.3. Summary of the 8051 Family of Components............1-4

1.4. References..........................................1-5

2. 8051 CROSS ASSEMBLER OVERVIEW............................2-1

2.1. Introduction........................................2-1

2.2. Symbols.............................................2-1

2.3. Labels..............................................2-2

2.4. Assembler Controls..................................2-3

2.5. Assembler Directives................................2-3

2.6. 8051 Instruction Mnemonics..........................2-4

2.7. Bit Addressing......................................2-6

2.8. ASCII Literals......................................2-6

2.9. Comments............................................2-6

2.10. The Location Counter...............................2-7

2.11. Syntax Summary.....................................2-7

2.12. Numbers and Operators..............................2-7

2.13. Source File Listing...............................2-10

2.14. Object File.......................................2-11

3. RUNNING THE 8051 CROSS ASSEMBLER.........................3-1

3.1. Cross Assembler Files...............................3-1

3.2. Minimum System Requirements.........................3-1

3.3. Running the Cross Assembler.........................3-1

3.4. Example Running the Cross Assembler.................3-3

3.5. DOS Hints and Suggestions...........................3-3

3.6. References..........................................3-4

4. 8051 INSTRUCTION SET.....................................4-1

4.1. Notation............................................4-1

4.2. 8051 Instruction Set Summary........................4-4

4.3. Notes..............................................4-14

4.4. References.........................................4-14

5. 8051 CROSS ASSEMBLER DIRECTIVES..........................5-1

5.1. Introduction........................................5-1

5.2. Symbol Definition Directives........................5-1

5.3. Segment Selection Directives........................5-4

5.4. Memory Reservation and Storage Directives...........5-5

5.5. Miscellaneous Directives............................5-7

5.6. Conditional Assembly Directives.....................5-8

6. 8051 CROSS ASSEMBLER CONTROLS............................6-1

6.1. Introduction........................................6-1

6.2. Assembler Control Descriptions......................6-1

7. 8051 CROSS ASSEMBLER MACRO PROCESSOR.....................7-1

7.1. Introduction........................................7-1

7.2. Macro Definition....................................7-1

7.3. Special Macro Operators.............................7-4

7.4. Using Macros........................................7-4

7.4.1. NESTING MACROS...............................7-4

7.4.2. LABELS IN MACROS.............................7-6

8. 8051 CROSS ASSEMBLER ERROR CODES.........................8-1

8.1. Introduction........................................8-1

8.2. Explanation of Error Messages.......................8-2

Appendices

A. SAMPLE PROGRAM AND LISTING...............................A-1

A.1. Source File.........................................A-1

A.2. Source File Listing.................................A-4

B. PRE-DEFINED BYTE AND BIT ADDRESSES.......................B-1

B.1. Pre-defined Byte Addresses..........................B-1

B.2. Pre-defined Bit Addresses..........................B-11

C. RESERVED SYMBOLS.........................................C-1

D. CROSS ASSEMBLER CHARACTER SET............................D-1

CHAPTER 1

8051 OVERVIEW

1.1. Introduction

For new users of MetaLink's ASM51 Cross Assembler, please take a

moment to fill out and return postage-prepaid User Registration

Card.  This card will be found with the system diskette in the

vinyl jacket at the front of this manual.  MetaLink will use this

information to send out, free of charge, any software updates

occurring during the warranty period.  Respondents will also be

notified of any new products or product enhancements.

The 8051 series of microcontrollers are highly integrated single

chip microcomputers with an 8-bit CPU, memory, interrupt

controller, timers, serial I/O and digital I/O on a single piece

of silicon. The current members of the 8051 family of components

include:

80C152JA/JB/JC/JD, 83C152JA/JC, 80C157

80C154, 83C154, 85C154

8044, 8344, 8744

80C451, 83C451, 87C451

80C452, 83C452, 87C452

8051, 8031, 8751, 80C51, 80C31, 87C51

80512, 80532

80515, 80535, 80C535, 80C515

80C517, 80C537

80C51FA, 83C51FA, 87C51FA, 83C51FB, 87C51FB, 83C51FC, 87C51FC

8052, 8032, 8752

80C321, 80C521, 87C521, 80C541, 87C541

8053, 9761, 8753

80C552, 83C552, 87C552

80C652, 83C652, 87C652

83C654, 87C654

83C751, 87C751

83C752, 87C752

80C851, 83C851

All members of the 8051 series of microcontrollers share a common

architecture.  They all have the same instruction set, addressing

modes, addressing range and memory spaces.  The primary

differences between different 8051 based products are the amount

of memory on chip, the amount and types of I/O and peripheral

functions, and the component's technology (see Table 1-1).

In the brief summary of the 8051 architecture that follows, the

term 8051 is used to mean collectively all available members of

the 8051 family.  Please refer to reference (1) for a complete

description of the 8051 architecture and the specifications for

all the currently available 8051 based products.

1-1

1.2. 8051 Architecture

The 8051 is an 8-bit machine.  Its memory is organized in bytes

and practically all its instruction deal with byte quantities.

It uses an Accumulator as the primary register for instruction

results. Other operands can be accessed using one of the four

different addressing modes available:  register implicit, direct,

indirect or immediate.  Operands reside in one of the five memory

spaces of the 8051.

The five memory spaces of the 8051 are: Program Memory, External

Data Memory, Internal Data Memory, Special Function Registers and

Bit Memory.

The Program Memory space contains all the instructions, immediate

data and constant tables and strings.  It is principally

addressed by the 16-bit Program Counter (PC), but it can also be

accessed by a few instructions using the 16-bit Data Pointer

(DPTR).  The maximum size of the Program Memory space is 64K

bytes.  Several 8051 family members integrate on-chip some amount

of either masked programmed ROM or EPROM as part of this memory

space (refer to Table 1-1).

The External Data Memory space contains all the variables,

buffers and data structures that can't fit on-chip.  It is

principally addressed by the 16-bit Data Pointer (DPTR), although

the first two general purpose register (R0,R1) of the currently

selected register bank can access a 256-byte bank of External

Data Memory.  The maximum size of the External Data Memory space

is 64Kbytes.  External  data memory can only be accessed using

the indirect addressing mode with the DPTR, R0 or R1.

The Internal Data Memory space is functionally the most important

data memory space.  In it resides up to four banks of general

purpose registers, the program stack, 128 bits of the 256-bit

memory, and all the variables and data structures that are

operated on directly by the program.  The maximum size of the

Internal Data Memory  space is 256-bytes.  However, different

8051 family members integrate different amounts of this memory

space on chip (see Amnt of RAM in Table 1-1).  The register

implicit, indirect and direct addressing modes can be used in

different parts of the Internal Data Memory space.

The Special Function Register space contains all the on-chip

peripheral I/O registers as well as particular registers that

need program access.  These registers include the Stack Pointer,

the PSW and the Accumulator. The maximum number of Special

Function Registers (SFRs) is 128, though the actual number on a

particular 8051 family member depends on the number and type of

peripheral functions integrated on-chip (see Table 1-1).  The

SFRs all have addresses greater than 127 and overlap the address

space of the upper 128 bytes of the Internal Data Memory space.

The two memory spaces are differentiated by addressing mode.  The

SFRs can only be accessed using the Direct addressing mode while

the upper 128 bytes of the Internal Data Memory (if integrated

on-chip) can only be accessed using the Indirect addressing mode.

1-2

The Bit Memory space is used for storing bit variables and flags.

There are specific instructions in the 8051 that operate only in

the Bit Memory space.  The maximum size of the Bit Memory space

is 256-bits.  128 of the bits overlap with 16-bytes of the

Internal Data Memory space and 128 of the bits overlap with 16

Special Function Registers.  Bits can only be accessed using the

bit instructions and the Direct addressing mode.

The 8051 has a fairly  complete set of arithmetic and logical

instructions.  It includes an 8X8 multiply and an 8/8 divide.

The 8051 is particularly good at processing bits (sometimes

called Boolean Processing).  Using the Carry Flag in the PSW as a

single bit accumulator, the 8051 can move and do logical

operations between the Bit Memory space and the Carry Flag.  Bits

in the Bit Memory space can also be used as general purpose flags

for the test bit and jump instructions.

1-3

Except for the MOVE instruction, the 8051 instructions can only

operate on either the Internal Data Memory space or the Special

Function Registers.  The MOVE instruction operates in all memory

spaces, including the External Memory space and Program Memory

space.

Program control instructions include the usual unconditional

calls and jumps as well as conditional relative jumps based on

the Carry Flag, the Accumulator's zero state, and the state of

any bit in the Bit Memory space.  Also available is a Compare and

Jump if Not Equal instruction and a Decrement Counter and Jump if

Not Zero loop instruction.  See Chapter 4 for a description of

the complete 8051 instruction set.

1.3. Summary of the 8051 Family of Components

1-4

Table 1-1: 8051 Family of Components

1.4. References

1. Intel Corp., 8-Bit Embedded Controllers, 1990.

2. Siemens Corp., Microcontroller Component 80515, 1985.

3. AMD Corp., Eight-Bit 80C51 Embedded Processors, 1990.

4. Signetics Corp., Microcontroller Users' Guide, 1989.

1-5

CHAPTER 2

8051 CROSS ASSEMBLER OVERVIEW

2.1. Introduction

The 8051 Cross Assembler takes an assembly language source file

created with a text editor and translates it into a machine

language object file. This translation process is done in two

passes over the source file.  During the first pass, the Cross

Assembler builds a symbol table from the symbols and labels used

in the source file.  It's during the second pass that the Cross

Assembler actually translates the source file into the machine

language object file.  It is also during the second pass that the

listing is generated.

The following is a discussion of the syntax required by the Cross

Assembler to generate error free assemblies.

2.2. Symbols

Symbols are alphanumeric representations of numeric constants,

addresses, macros, etc.  The legal character set for symbols is

the set of letters, both upper and lower case (A..Z,a..z), the

set of decimal numbers (0..9) and the special characters,

question mark (?) and underscore (_).  To ensure that the Cross

Assembler can distinguish between a symbol and a number, all

symbols must start with either a letter or special character (?

or _).  The following are examples of legal symbols:

PI

Serial_Port_Buffer

LOC_4096

?_?_?

In using a symbol, the Cross Assembler converts all letters to

upper case. As a result, the Cross Assembler makes no distinction

between upper and lower case letters.  For example, the following

two symbols would be seen as the same symbol by the Cross

Assembler:

Serial_Port_Buffer

SERIAL_PORT_BUFFER

Symbols can be defined only once.  Symbols can be up to 255

characters in length, though only the first 32 are significant.

Therefore, for symbols to be unique, they must have a unique

character pattern within the first 32 characters.  In the

following example, the first two symbols would be seen by the

Cross Assembler as duplicate symbols, while the third and fourth

2-1

symbols are unique.

BEGINNING_ADDRESS_OF_CONSTANT_TABLE_1

BEGINNING_ADDRESS_OF_CONSTANT_TABLE_2

CONSTANT_TABLE_1_BEGINNING_ADDRESS

CONSTANT_TABLE_2_BEGINNING_ADDRESS

There are certain symbols that are reserved and can't be defined

by the user.  These reserved symbols are listed in Appendix C and

include the assembler directives, the 8051 instruction mnemonics,

implicit operand symbols, and the following assembly time

operators that have alphanumeric symbols: EQ, NE, GT, GE, LT, LE,

HIGH, LOW, MOD, SHR, SHL, NOT,

AND, OR and XOR.

The reserved implicit operands include the symbols A, AB, C,

DPTR, PC, R0, R1, R2, R3, R4, R5, R6, R7, AR0, AR1, AR2, AR3,

AR4, AR5, AR6 and AR7.  These symbols are used primarily as

instruction operands.  Except for AB, C,  DPTR or PC, these

symbols can also be used to define other symbols (see EQU

directive in Chapter 5).

The following are examples of illegal symbols with an explanation

of why they are illegal:

1ST_VARIABLE        (Symbols can not start with a number.)

ALPHA#              (Illegal character "#" in symbol.)

MOV                 (8051 instruction mnemonic)

LOW                 (Assembly operator)

DATA                (Assembly directive)

2.3. Labels

Labels are special cases of symbols.  Labels are used only before

statements that have physical addresses associated with them.

Examples of such statements are assembly language instructions,

data storage directives (DB and DW), and data reservation

directives (DS and DBIT).  Labels must follow all the rules of

symbol creation with the additional requirement that they be

followed by a colon.  The following are legal examples of label

uses:

TABLE_OF_CONTROL_CONSTANTS:

DB    0,1,2,3,4,5   (Data storage)

MESSAGE:        DB    'HELP'        (Data storage)

VARIABLES:      DS    10            (Data reservation)

BIT_VARIABLES:  DBIT  16            (Data reservation)

START:          MOV   A,#23         (Assembly language instruction)

2.4. Assembler Controls

2-2

Assembler controls are used to control where the Cross Assembler

gets its input source file, where it puts the object file, and

how it formats the listing file.  Table 2-1 summarizes the

assembler controls available.  Refer to Chapter 6 for a detailed

explanation of the controls.

Table 2-1: Summary of Cross Assembler Controls

As can be seen in Table 2-1, all assembler controls are prefaced

with a dollar sign ($). No spaces or tabs are allowed between the

dollar sign and the body of the control.  Also, only one control

per line is permitted.  However, comments can be on the same line

as a control.  The following are examples of assembler controls:

$TITLE(8051 Program Ver. 1.0)

$LIST

$PAGEWIDTH(132)

2.5. Assembler Directives

Assembler directives are used to define symbols, reserve memory

space, store values in program memory and switch between

different memory spaces.  There are also directives that set the

location counter for the active segment and identify the end of

the source file.  Table 2-2 summarizes the assembler directives

available.  These directives are fully explained in Chapter 5.

Table 2-2: Summary of Cross Assembler Directives

Only one directive per line is allowed, however comments may be

2-3

included.  The following are examples of assembler directives:

TEN       EQU       10

RESET     CODE      0

ORG       4096

2.6. 8051 Instruction Mnemonics

The standard 8051 Assembly Language Instruction mnemonics plus

the generic CALL and JMP instructions are recognized by the Cross

Assembler and are summarized in Table 2-3. See Chapter 4 for the

operation of the individual instructions.

2-4

Table 2-3: 8051 Instructions and Mnemonics

When the Cross Assembler sees a generic CALL or JMP instruction,

it will try to translate the instruction into its most byte

efficient form.  The Cross Assembler will translate a CALL into

one of two instructions (ACALL or LCALL) and it will translate a

generic JMP into one of three instructions (SJMP, AJMP or LJMP).

The choice of instructions is based on which one is most byte

efficient.  The generic CALL or JMP instructions saves the

programmer the trouble of determining which form is best.

However, generic CALLs and JMPs do have their limitations.  While

the byte efficiency algorithm works well for previously defined

locations, when the target location of the CALL or JMP is a forward

location (a location later on in the program), the assembler has no

way of determining the best form of the instruction.  In this case

the Cross Assembler simply puts in the long version (LCALL or LJMP)

of the instruction, which may not be the most byte efficient.  NOTE

that the generic CALLs and JMPs must not be used for the 751/752

device as LCALL and LJMP are not legal instructions for those

devices.  Instead use ACALL and AJMP explicitly.

For instructions that have operands, the operands must be

separated from the mnemonic by at least one space or tab.  For

instructions that have multiple operands, each operand must be

separated from the others by a comma.

Two addressing modes require the operands to be preceded by

special symbols to designate the addressing mode.  The AT sign

(@) is used to designate the indirect addressing mode.  It is

used primarily with Register 0 and Register 1 (R0, R1), but is

can also be used with the DPTR in the MOVX and the Accumulator in

MOVC and JMP @A+DPTR instructions.  The POUND sign (#) is used to

designate an immediate  operand.  It can be used to preface

either a number or a symbol representing a number.

A third symbol used with the operands actually specifies an

operation.  The SLASH (/) is used to specify that the contents of

a particular bit address is to be complemented before the

2-5

instruction operation.  This is used with the ANL and ORL bit

instructions.

Only one assembly language instruction is allowed per line.

Comments are allowed on the same line as an instruction, but only

after all operands have been specified.  The following are

examples of instruction statements:

START:    LJMP      INIT

MOV       @R0,Serial_Port_Buffer

CJNE      R0 , #TEN, INC_TEN

ANL       C,/START_FLAG

CALL      GET_BYTE

RET

2.7. Bit Addressing

The period (.) has special meaning to the Cross Assembler when

used in a symbol.  It is used to explicitly specify a bit in a

bit-addressable symbol.  For example, it you wanted to specify

the most significant bit in the Accumulator, you could write

ACC.7, where ACC was previously defined as the Accumulator

address.  The same bit can also be selected using the physical

address of the byte it's in.  For example, the Accumulator's

physical address is 224.  The most significant bit of the

Accumulator can be selected by specifying 224.7.  If the symbol

ON was defined to be equal to the value 7, you could also specify

the same bit by either ACC.ON or 224.ON.

2.8. ASCII Literals

Printable characters from the ASCII character set can be used

directly as an immediate operand, or they can used to define

symbols or store ASCII bytes in Program Memory.  Such use of the

ASCII character set is called ASCII literals.  ASCII literals are

identified by the apostrophe (') delimiter.  The apostrophe

itself can be used as an ASCII literal.  In this case, use two

apostrophes in a row.  Below are examples of using ASCII

literals.

MOV  A,#'m'    ;Load A with 06DH (ASCII m)

QUOTE     EQU  ''''      ;QUOTE defined as 27H (ASCII single quote)

DB   '8051'    ;Store in Program Memory

2.9. Comments

Comments are user defined character strings that are not

processed by the Cross Assembler.  A comment begins with a

semicolon ( ; )  and ends at the carriage return/line feed pair

that terminates the line.  A comment can appear anywhere in a

line, but it has to be the last field.  The following are

examples of comment lines:

2-6

; Begin initialization routine here

$TITLE(8051 Program Vers. 1.0)          ;Place version number here

TEN       EQU       10                  ;Constant

; Comment can begin anywhere in a line

MOV  A,Serial_Port_Buffer     ; Get character

2.10. The Location Counter

The Cross Assembler keeps a location counter for each of the five

segments (code, internal data, external data, indirect internal

data and bit data).  Each location counter is initialized to zero

and can be modified using Assembler Directives described in

Chapter 5.

The dollar sign ($) can be used to specify the current value of

the location counter of the active segment.  The following are

examples of how this can be used:

JNB  FLAG,$        ;Jump on self until flag is reset

CPYRGHT:  DB   'Copyright, 1983'

CPYRGHT_LENGTH

EQU  $-CPYRGHT-1   ;Calculate length of copyright message

2.11. Syntax Summary

Since the Cross Assembler essentially translates the source file

on a line by line basis, certain rules must be followed to ensure

the translation process is done correctly.  First of all, since

the Cross Assembler's line buffer is 256 characters deep, there

must always be a carriage return/line feed pair within the first

256 columns of the line.

A legal source file line must begin with either a control, a

symbol, a label, an instruction mnemonic, a directive, a comment

or it can be null (just the carriage return/line feed pair).  Any

other beginning to a line will be flagged as an error.

While a legal source file line must begin with one of the above

items, the item doesn't have to begin in the first column of the

line.  It only must be the first field of the line.  Any number

(including zero) of spaces or tabs, up to the maximum line size,

may precede it.

Comments can be placed anywhere, but they must be the last field

in any line.

2.12. Numbers and Operators

The Cross Assembler accepts numbers in any one of four radices:

binary, octal, decimal and hexadecimal.  To specify a number in a

specific radix, the number must use the correct digits for the

particular radix and immediately following the number with its

2-7

radix designator.  Decimal is the default radix and the use of

its designator is optional.  An hexadecimal number that would

begin with a letter digit must be preceded by a 0 (zero) to

distinguish it from a symbol. The internal representation of

numbers is 16-bits, which limits the maximum number possible.

Table 2-4 summarizes the radices available.

MAXIMUM LEGAL

RADIX          DESIGNATOR      LEGAL DIGITS            NUMBER

-----------    ----------      ------------     -----------------

Binary             B           0,1              1111111111111111B

Octal             O,Q          0,1,2,3,4,5,          177777O

6,7                   177777Q

Decimal        D,(default)     0,1,2,3,4,5,           65535D

6,7,8,9                65535

Hexadecimal        H           0,1,2,3,4,5,           0FFFFH

6,7,8,9,A,B,

C,D,E,F

Table 2-4: Cross Assembler Radices

No spaces or tabs are allowed between the number and the radix

designator.  The letter digits and radix designators can be in

upper or lower case.  The following examples list the decimal

number 2957 in each of the available radices:

101110001101B            (Binary)

5615o or 5615Q           (Octal)

2957 or 2957D            (Decimal)

0B8DH, 0b8dh             (Hexadecimal)

When using radices with explicit bit symbols, the radix

designator follows the byte portion of the address as shown in

the following examples:

0E0H.7         Bit seven of hexadecimal address 0E0

200Q.ON        Bit ON of octal address 200

The Cross Assembler also allows assembly time evaluation of

arithmetic expressions up to thirty-two levels of embedded

parentheses.  All calculations use integer numbers and are done

in sixteen bit precision.

OPERATOR SYMBOL          OPERATION

---------------          ------------------------

+                  Addition

Unary positive

-                  Subtraction

Unary negation (2's complement)

*                  Multiplication

/                  Integer division (no remainder)

MOD                 Modulus (remainder of integer division)

SHR                 Shift right

SHL                 Shift left

2-8

NOT                 Logical negation (1's complement)

AND                 Logical and

OR                  Inclusive or

XOR                 Exclusive or

LOW                 Low order 8-bits

HIGH                High order 8-bits

EQ, =               Relational equal

NE, <>              Relational not equal

GT, >               Relational greater than

GE, >=              Relational greater than or equal

LT, <               Relational less than

LE, <=              Relational less than or equal

( )                 Parenthetical statement

Table 2-5: Assembly Time Operations

The relational operators test the specified values and return

either a True or False.  False is represented by a zero value,

True is represented by a non zero value (the True condition

actually returns a 16-bit value with every bit set; i.e.,

0FFFFH).  The relational operators are used primarily with the

Conditional Assembly capability of the Cross Assembler.

Table 2-5 lists the operations available while Table 2-6 lists

the operations precedence in descending order.  Operations with

higher precedence are done first.  Operations with equal

precedence are evaluated from left to right.

OPERATION                PRECEDENCE

---------                ----------

(,)                   HIGHEST

HIGH,LOW

*,/,MOD,SHR,SHL

+,-

EQ,LT,GT,LE,GE,NE,=,<,>,<=,>=,<>

NOT

AND

OR,XOR                   LOWEST

Table 2-6: Operators Precedence

The following are examples of all the available operations and

their result:

HIGH(0AADDH)        will return a result of 0AAH

LOW(0AADDH)         will return a result of 0DDH

7*4                 will return a result of 28

7/4                 will return a result of 1

7 MOD 4             will return a result of 3

1000B SHR 2         will return a result of 0010B

1010B SHL 2         will return a result of 101000B

10+5                will return a result of 15

+72                 will return a result of 72

25-17               will return a result of 8

2-9

-1                  will return a result of 1111111111111111B

NOT 1               will return a result of 1111111111111110B

7 EQ 4, 7 = 4       will return a result of 0

7 LT 4, 7 < 4       will return a result of 0

7 GT 4, 7 > 4       will return a result of 0FFFFH

7 LE 4, 7 <= 4      will return a result of 0

7 GE 4, 7 >= 4      will return a result of 0FFFFH

7 NE 4, 7 <> 4      will return a result of 0FFFFH

1101B AND 0101B     will return a result of 0101B

1101B OR 0101B      will return a result of 1101B

1101B XOR 0101B     will return a result of 1000B

2.13. Source File Listing

The source file listing displays the results of the Cross

Assembler translation.  Every line of the listing includes a copy

of the original source line as well as a line number and the

Cross Assembler translation.

For example, in translating the following line taken from the

middle of a source file:

TRANS:    MOV  R7,#32         ;Set up pointer

the listing will print:

002F 7920         152   TRANS:  MOV   R1,#32        ;Set up pointer

The '002F' is the current value of the location counter in

hexadecimal.   The '7920' is the translated instruction, also in

hexadecimal.  The '152' is the decimal line number of the current

assembly.  After the line number is a copy of the source file

line that was translated.

Another example of a line in the listing file is as follows:

015B 13      =1 267 +2          RRC   A

Here we see two additional fields.  The '=1' before the line

number gives the current nesting of include files.  The '+2'

after the line number gives the current macro nesting.  This line

essentially says that this line comes from a second level nesting

of a macro that is part of an include file.

Another line format that is used in the listing is that of symbol

definition.  In this case the location counter value and

translated instruction fields described above are replaced with

the definition of the symbol.  The following are examples of

this:

00FF             67   MAX_NUM      EQU       255

REG              68   COUNTER      EQU       R7

The '00FF' is the hexadecimal value of the symbol MAX_NUM.

Again, '67'is the decimal line number of the source file and the

2-10

remainder of the first line is a copy of the source file.  In the

second line above, the 'REG' shows that the symbol COUNTER was

defined to be a general purpose register.

Optionally, a listing can have a page header that includes the

name of the file being assembled, title of program, date and page

number.  The header and its fields are controlled by specific

Assembler Controls (see Chapter 6).

The default case is for a listing to be output as a file on the

default drive with the same name as the entered source file and

an extension of .LST.  For example, if the source file name was

PROGRAM.ASM, the listing file would be called PROGRAM.LST.  Or if

the source file was called MODULE1, the listing file would be

stored as MODULE1.LST.  The default can be changed using the

$NOPRINT and $PRINT() Assembler Controls (see Chapter 6).

2.14. Object File

The 8051 Cross Assembler also creates a machine language object

file.  The format of the object file is standard Intel

Hexadecimal.  This Hexadeciaml file can be used to either program

EPROMs using standard PROM Programmers for prototyping, or used

to pattern masked ROMs for production.

The default case is for the object file to be output on the

default drive with the same name as the first source file and an

extension of .HEX.  For example, if the source file name was

PROGRAM.ASM, the object file would be called PROGRAM.HEX.  Or if

the source file was called MODULE1, the object file would be

stored as MODULE1.HEX.  The default can be changed using the

$NOOBJECT and $OBJECT() Assembler Controls (see Chapter 6).

2-11

CHAPTER 3

RUNNING THE 8051 CROSS ASSEMBLER ON PC-DOS/MS-DOS SYSTEMS

3.1. Cross Assembler Files

The floppy disk you receive with this manual is an 8 sector,

single-sided, double density disk.  This distribution disk will

contain the following files:

ASM51.EXE           The Cross Assembler program itself

MOD152              Source file for the $MOD152 control

MOD154              Source file for the $MOD154 control

MOD252              Source file for the $MOD252 control

MOD44               Source file for the $MOD44 control

MOD451              Source file for the $MOD451 control

MOD452              Source file for the $MOD452 control

MOD51               Source file for the $MOD51 control

MOD512              Source file for the $MOD512 control

MOD515              Source file for the $MOD515 control

MOD517              Source file for the $MOD517 control

MOD52               Source file for the $MOD52 control

MOD521              Source file for the $MOD521 control

MOD552              Source file for the $MOD552 control

MOD652              Source file for the $MOD652 control

MOD751              Source file for the $MOD751 control

MOD752              Source file for the $MOD752 control

MOD851              Source file for the $MOD851 control

There will also be one or more files with an extension of .ASM.

These are sample programs.  Listings of these programs can be

found in Appendix A.

DON'T USE THE DISTRIBUTION DISK.  MAKE WORKING AND BACKUP COPIES

FROM THE DISTRIBUTION DISK AND THEN STORE THE DISTRIBUTION DISK

IN A SAFE PLACE.

3.2. Minimum System Requirements

With DOS 2.0 or later - 96K RAM

1 Floppy Disk Drive

3.3. Running the Cross Assembler

Once you've created an 8051 assembly language source text file in

accordance with the guidelines in Chapter 2, you are now ready to

run the Cross Assembler.  Make sure your system is booted and the

DOS prompt ( A> ) appears on the screen.  Place the disk with the

8051 Cross Assembler on it in the drive and simply type (in all

the following examples, the symbol <CR> is used to show where the

3-1

ENTER key was hit):

ASM51<CR>

If the 8051 Cross Assembler disk was placed in a drive other than

the default drive, the drive name would have to be typed first.

For example, if the A drive is the default drive, and the 8051

Cross Assembler is in the B drive, you would then type:

B:ASM51<CR>

After loading the program from the disk, the program's name, its

version number and general copyright information will be dis-

played on the screen.  The Cross Assembler then asks for the

source file name to begin the assembly process.

Source file drive and name [.ASM]:

At this point, if you have only one floppy disk drive and the

8051 Cross Assembler and source files are on separate disks,

remove the disk with the 8051 Cross Assembler on it and replace

it with your source file disk.

Next, enter the source file name.  If no extension is given, the

Cross Assembler will assume an extension of .ASM.  If no drive is

given, the Cross Assembler will assume the default drive.  Since

in every case where no drive is given, the Cross Assembler

assumes the default drive, it is generally a good practice to

change the default drive to the drive with your source files.

An alternative method for entering the source file is in the

command line.  In this case, after typing in ASM51, type in a

space and the source file name (again if no extension is given,

source file on the command line:

A>ASM51 B:CONTROL.A51<CR>

After the source file name has been accepted, the Cross Assembler

will begin the translation process.  As it starts the first pass

of its two pass process, it will print on the screen:

First pass

At the completion of the first pass, and as it starts its second

pass through the source file, the Cross Assembler will display:

Second pass

When second pass is completed, the translation process is done

and the Cross Assembler will print the following message:

ASSEMBLY COMPLETE, XX ERRORS FOUND

XX is replaced with the actual number of errors that were found.

Disk I/O may continue for a while as the Cross Assembler appends

3-2

the symbol table to the listing file.

3.4. Example Running the Cross Assembler

The following is an example of an actual run.  The Cross

Assembler will take the source file SAMPLE.ASM from Drive A

(default drive).

Again, the symbol <CR> is used to show where the ENTER key was

hit.

A>ASM51<CR>

8 0 5 1    C R O S S    A S S E M B L E R

Version  1.2

(c) Copyright 1984, 1985, 1986, 1987, 1988, 1989, 1990

MetaLink Corporation

Source file drive and name [.ASM]: sample<CR>

First pass

Second pass

ASSEMBLY COMPLETE, 0 ERRORS FOUND

3.5. DOS Hints and Suggestions

If you are using DOS 2.0 or later, you may want to use the BREAK

ON command before you run the Cross Assembler.  This will allow

you to abort (Ctrl-Break) the Cross Assembler at any time.

Otherwise, you will only be able to abort the Cross Assembler

after it completes a pass through the source file.  If you are

assembling a large file, this could cause you a several minute

wait before the Cross Assembler aborts.

The reason for this it that the  default condition  for DOS  to

recognizes a Ctrl-Break is when the program (in this case the

Cross Assembler) does keyboard, screen or printer I/O.

Unfortunately, the assembler does this very rarely (once each

pass).  By using the BREAK ON command, DOS will recognize a Ctrl-

Break for all I/O, including disk I/O.  Since the Cross Assembler

is constantly doing disk I/O, with BREAK ON you can abort almost

immediately by hitting the Ctrl-Break keys.

3-3

So much for the good news.  However, aborting a program can cause

some undesirable side-effects.  Aborting a program while files

are open causes DOS to drop some information about the open

files.  This results in disk sectors being allocated when they

are actually free.  Your total available disk storage shrinks.

You should make the practice of running CHKDSK with the /F switch

periodically to recover these sectors.

The Cross Assembler run under DOS 2.0 or later supports

redirection. You can specify the redirection on the command line.

Use the following form:

ASM51 <infile >outfile

"infile" and "outfile" can be any legal file designator. The

Cross Assembler will take its input from the "infile" instead of

the keyboard and will send its output to "outfile" instead of the

screen.

Note that redirection of input in ASM51 is redundant since the

assembler is an absolute assembler and has no command line

options other than the file name argument.

Output redirection is useful for speeding up the assembly

process.  Because assembly-time errors are directed to std_err in

DOS, an error listing cannot be redirected to a file

To make the .lst file serve as an error-only file, use the Cross

Assembler Controls $PRINT (create a list file) $NOLIST (turn the

listing off).  Use the Cross Assembler Controls $NOSYMBOLS to

further compress the error-only listing resulting from the

manipulation of the list file controls.  See Chapter 6 for more

information.  The errors will be listed in the .lst file, as

usual.

If the control $NOPRINT (see Chapter 6) is active, all error

messages are send to the screen.

3.6. References

1. IBM Corp., Disk Operating System, Version 1.10, May 1982.

2. IBM Corp., Disk Operating System, Version 2.00, January 1983.

3-4

CHAPTER 4

8051 INSTRUCTION SET

4.1. Notation

Below is an explanation of the column headings and column

contents of the 8051 Instruction Set Summary Table that follows

in this chapter.

MNEMONIC

The MNEMONIC column contains the 8051 Instruction Set Mnemonic

and a brief description of the instruction's operation.

OPERATION

The OPERATION column describes the 8051 Instruction Set in unam-

biguous symbology.  Following are the definitions of the symbols

used in this column.

<n:m>                  Bits of a register inclusive.  For

example, PC<10:0> means bits 0 through 10

inclusive of the PC. Bit 0 is always the

least significant bit.

+                    Binary addition

-                    Binary 2s complement subtraction

/                    Unsigned integer division

X                    Unsigned integer multiplication

~                    Binary complement (1s complement)

^                    Logical And

v                    Inclusive Or

v                    Exclusive Or

>                    Greater than

<>                    Not equal to

=                    Equals

->                    Is written into.  For example, A + SOper -

> A means the result of the binary

addition between A and the Source Operand

is written into A.

A                    The 8-bit Accumulator Register.

AC                    The Auxiliary Carry Flag in the Program

Status Word

CF                    The Carry Flag in the Program Status Word

DOper                  The Destination Operand used in the

instruction.

DPTR                   16-bit Data Pointer

Intrupt Active Flag    Internal Flag that holds off interrupts

4-1

until the Flag is cleared.

Jump Relative to PC    A Jump that can range between -128 bytes

and +127 bytes from the PC value of

the next instruction.

Paddr                  A 16-bit Program Memory address

PC                    The 8051 Program Counter. This 16-bit

register points to the byte in the

Program Memory space that is fetched as

part of the instruction stream.

PM(addr)               Byte in Program Memory space pointed

to by addr.

Remainder              Integer remainder of unsigned integer division

SOper                  The Source Operand used in the instruction.

SP                    8-bit Stack Pointer

STACK                  The Last In First Out data structure that

is controlled by the 8-bit Stack

Pointer (SP).  Sixteen bit quantities are

pushed on the stack low byte first.

DEST ADDR MODE/SOURCE ADDR MODE

These two columns specify the Destination and Source Addressing

Modes, respectively, that are available for each instruction.

AB             The Accumulator-B Register pair.

Accumulator    Operand resides in the accumulator

Bit Direct     Operand is the state of the bit specified by the

Bit Memory address.

Carry Flag     Operand is the state of the 1-bit Carry flag in

the Program Status Word (PSW).

Data Pointer   Operand resides in the 16-bit Data Pointer

Register.

Direct         Operand is the contents of the specified 8-bit

Internal Data Memory address from 0

(00H) to 127 (7FH) or a Special Function Register

address.

Indirect       Operand is the contents of the address contained

in the register specified.

Immediate      Operand is the next sequential byte after the

instruction in Program Memory space

Prog Direct    16-bit address in Program Memory Space.

Prog Indir     Operand in Program Memory Space is the address

contained in the register specified.

Register       Operand is the contents of the register specified.

Stack          Operand is on the top of the Stack.

ASSEMBLY LANGUAGE FORM

This column contains the correct format of the instructions that

are recognized by the Cross Assembler.

A         Accumulator

AB        Accumulator-B Register pair.

4-2

C         Carry Flag

Baddr     Bit Memory Direct Address.

Daddr     Internal Data Memory or Special Function Register

Direct Address.

data      8-bit constant data.

data16    16-bit constant data.

DPTR      16-bit Data Pointer Register.

PC        16-bit Program Counter.

Paddr     16-bit Program Memory address

Ri        Indirect Register. R0 or R1 are the only indirect

registers.

Roff      8-bit offset for Relative Jump.

Rn        Implicit Register.  Each register bank has 8 general

purpose registers, designated R0, R1, R2, R3,

R4, R5, R6, R7.

HEX OPCODE

This column gives the machine language hexadecimal opcode for

each 8051 instruction.

BYT

This column gives the number of bytes in each 8051 instruction.

CYC

This column gives the number of cycles of each 8051 instruction.

The time value of a cycle is defined as 12 divided by the

oscillator frequency. For example, if running an 8051 family

component at 12 MHz, each cycle takes 1 microsecond.

PSW

This column identifies which condition code flags are affected by

the operation of the individual instructions.  The condition code

flags available on the 8051 are the Carry Flag, CF, the Auxiliary

Carry Flag, AC, and the Overflow Flag, OV.

It should be noted that the PSW is both byte and bit directly

addressable.  Should the PSW be the operand of an instruction

that modifies it, the condition codes could be changed even if

this column states that the instruction doesn't affect them.

0        Condition code is cleared

1        Condition code is set

*        Condition code is modified by instruction

-        Condition code is not affected by instruction

4.2. 8051 Instruction Set Summary

4-3

4-4

4-5

4-6

4-7

4-8

4-9

4-10

4-11

4-12

4.3. Notes

1       There are 8 possible opcodes.  Starting with 11H as the

opcode base, the final opcode is formed by placing bits 8, 9 and

10 of the target address in bits 5, 6 and 7 of the opcode.  The 8

possible opcodes in hexadecimal are then: 11, 31, 51, 71, 91, B1,

D1, F1.

2        There are 8 possible opcodes.  Starting with 01H as

the opcode base, the final opcode is formed by placing bits 8, 9

and 10 of the target address in bits 5, 6 and 7 of the opcode.

The 8 possible opcodes in hexadecimal are then: 01, 21, 41, 61,

81, A1, C1, E1.

3        The Carry Flag is set if the Destination Operand is

less than the Source Operand.  Otherwise the Carry Flag is

cleared.

4        The Carry Flag is set if the BCD result in the

Accumulator is greater than decimal 99.

5        The Overflow Flag is set if the B Register contains

zero (flags a divide by zero operation).  Otherwise the Overflow

Flag is cleared.

6         If any of the condition code flags are specified as

the operand of this instruction, they will be reset by the

instruction if they were originally set.

7         The high byte of the 16-bit product is placed in the

B Register, the low byte in Accumulator.

4.4. References

1. Intel Corp., Microcontroller Handbook, 1984.

4-13

CHAPTER 5

8051 CROSS ASSEMBLER DIRECTIVES

5.1. Introduction

The 8051 Cross Assembler Directives are used to define symbols,

reserve memory space, store values in program memory,  select

various memory spaces, set the current segment's location counter

and identify the end of the source file.

Only one directive per line is allowed, however comments may be

included.  The remaining part of this chapter details the

function of each directive.

5.2. Symbol Definition Directives

EQU Directive

The EQUate directive is used to assign a value to a symbol.  It

can also be used to specify user defined names for the  implicit

operand symbols predefined for the Accumulator (i.e., A) and the

eight General Purpose Registers (i.e., R0 thru R7).

The format for the EQU directive is: symbol, followed by one or

more spaces or tabs, followed by EQU, followed by one or more

spaces or tabs, followed by a number, arithmetic expression,

previously defined symbol (no forward references allowed) or one

of the allowed implicit operand symbols (e.g., A, R0, R1, R2, R3,

R4, R5, R6, R7), followed by an optional comment.

Below are examples of using the EQU Directive:

TEN        EQU     10      ;Symbol equated to a number

COUNTER    EQU     R7      ;User defined symbol for the implicit

;operand symbol R7.  COUNTER can now

;be used wherever it is legal to use

;R7.  For example the instruction

;INC R7 could now be written INC COUNTER.

ALSO_TEN   EQU     TEN     ;Symbol equated to a previously defined

;symbol.

FIVE       EQU     TEN/2   ;Symbol equated to an arithmetic exp.

A_REG      EQU     A       ;User defined symbol for the implicit

;operand symbol A.

ASCII_D    EQU     'D'     ;Symbol equated to an ASCII literal

SET Directive

Similar to the EQU directive, the SET directive is used to assign

a value or implicit operand to a user defined symbol.  The

difference however, is that with the EQU directive, a symbol can

5-1

only be defined once.  Any attempt to define the symbol again

will cause the Cross Assembler to flag it as an error.  On the

other hand, with the SET directive, symbols are redefineable.

There is no limit to the number of times a symbol can be

redefined with the SET directive.

The format for the SET directive is: symbol, followed by one or

more spaces or tabs, followed by SET, followed by one or more

spaces or tabs, followed by a number, arithmetic expression,

previously defined symbol (no forward references allowed) or one

of the allowed implicit operand symbols (e.g., A, R0, R1, R2, R3,

R4, R5, R6, R7), followed by an optional comment.

Below are examples of using the SET Directive:

POINTER   SET       R0          ;Symbol equated to register 0

POINTER   SET       R1          ;POINTER redefined to register 1

COUNTER   SET       1           ;Symbol initialized to 1

COUNTER   SET       COUNTER+1   ;An incrementing symbol

BIT Directive

The BIT Directive assigns an internal bit memory direct address

to the symbol.  If the numeric value of the address is between 0

and 127 decimal, it is a bit address mapped in the Internal

Memory Space.  If the numeric value of the address is between 128

and 255, it is an address of a bit located in one of the Special

Function Registers.  Addresses greater than 255 are illegal and

will be flagged as an error.

The format for the BIT Directive is: symbol, followed by one or

more spaces or tabs, followed by BIT, followed by one or more

spaces or tabs, followed by a number, arithmetic expression, or

previously defined symbol (no forward references allowed),

followed by an optional comment.

Below are examples of using the BIT Directive:

CF        BIT     0D7H         ;The single bit Carry Flag in PSW

OFF_FLAG  BIT     6            ;Memory address of single bit flag

ON_FLAG   BIT     OFF_FLAG+1   ;Next bit is another flag

CODE Directive

The CODE Directive assigns an address located in the Program

Memory Space to the symbol.  The numeric value of the address

cannot exceed 65535.

The format for the CODE Directive is: symbol, followed by one or

more spaces or tabs, followed by CODE, followed by one or more

spaces or tabs, followed by a number, arithmetic expression, or

previously defined symbol (no forward references allowed),

followed by an optional comment.

5-2

Below are examples of using the CODE Directive:

RESET     CODE      0

EXTI0     CODE      RESET + (1024/16)

DATA Directive

The DATA Directive assigns a directly addressable internal memory

address to the symbol.  If the numeric value of the address is

between 0 and 127 decimal, it is an address of an Internal Data

Memory location.  If the numeric value of the address is between

128 and 255, it is an address of a Special Function Register.

Addresses greater than 255 are illegal and will be flagged as an

error.

The format for the DATA Directive is: symbol, followed by one or

more spaces or tabs, followed by DATA, followed by one or more

spaces or tabs, followed by a number, arithmetic expression, or

previously defined symbol (no forward references allowed),

followed by an optional comment.

Below are examples of using the DATA Directive:

PSW         DATA      0D0H       ;Defining the Program Status address

BUFFER      DATA      32         ;Internal Data Memory address

FREE_SPAC   DATA      BUFFER+16  ;Arithmetic expression.

IDATA Directive

The IDATA Directive assigns an indirectly addressable internal

data memory address to the symbol.  The numeric value of the

address can be between 0 and 255 decimal.  Addresses greater than

255 are illegal and will be flagged as an error.

The format for the IDATA Directive is: symbol, followed by one or

more spaces or tabs, followed by IDATA, followed by one or more

spaces or tabs, followed by a number, arithmetic expression, or

previously defined symbol (no forward references allowed),

followed by an optional comment.

Below are examples of using the IDATA Directive:

TOKEN      IDATA     60

BYTE_CNT   IDATA     TOKEN + 1

ADDR       IDATA     TOKEN + 2

XDATA Directive

The XDATA Directive assigns an address located in the External

Data Memory Space to the symbol.  The numeric value of the

address cannot exceed 65535.

5-3

The format for the XDATA Directive is: symbol, followed by one or

more spaces or tabs, followed by XDATA, followed by one or more

spaces or tabs, followed by a number, arithmetic expression, or

previously defined symbol (no forward references allowed),

followed by an optional comment.

Below are examples of using the XDATA Directive:

USER_BASE      XDATA     2048

HOST_BASE      XDATA     USER_BASE + 1000H

5.3. Segment Selection Directives

There are five Segment Selection Directives: CSEG, BSEG, DSEG,

ISEG, XSEG, one for each of the five memory spaces in the 8051

architecture.  The CSEG Directive is used to select the Program

Memory Space.  The BSEG Directive is used to select the Bit

Memory Space.  The DSEG Directive is used to select the directly

addressable Internal Data Memory Space.  The ISEG is used to

select the indirectly addressable Internal Data Memory Space.

The XSEG is used to select the External Data Memory Space.

Each segment has its own location counter that is reset to zero

during the Cross Assembler program initialization.  The contents

of the location counter can be overridden by using the optional

AT after selecting the segment.

The Program Memory Space, or CSEG, is the default segment and is

selected when the Cross Assembler is run.

The format of the Segment Selection Directives are: zero or more

spaces or tabs, followed by the Segment Selection Directive,

followed by one or more spaces or tabs,  followed by the optional

segment location counter override AT command and value, followed

by an optional comment.

The value of the AT command can be a number, arithmetic

expression or previously defined symbol (forward references are

not allowed).  Care should be taken to ensure that the location

counter does not advance beyond the limit of the selected

segment.

Below are examples of the Segment Selection Directives:

DSEG                           ;Select direct data segment using

;current location counter value.

BSEG AT 32                     ;Select bit data segment forcing

;location counter to 32 decimal.

XSEG AT (USER_BASE * 5) MOD 16 ;Arithmetic expressions can be

;used to specify location.

5.4. Memory Reservation and Storage Directives

5-4

DS Directive

The DS Directive is used to reserve space in the currently

selected segment in byte units.  It can only be used when ISEG,

DSEG or XSEG are the currently active segments.  The location

counter of the segment is advanced by the value of the directive.

Care should be taken to ensure that the location counter does not

advance beyond the limit of the segment.

The format for the DS Directive is: optional label, followed by

one or more spaces or tabs, followed by DS, followed by one or

more spaces or tabs, followed by a number, arithmetic expression,

or previously defined symbol (no forward references allowed),

followed by an optional comment.

Below is an example of using the DS Directive in the internal

Data Segment.  If, for example, the Data Segment location counter

contained 48 decimal before the example below, it would contain

104 decimal after processing the example.

DSEG           ;Select the data segment

DS        32   ;Label is optional

SP_BUFFER:     DS        16   ;Reserve a buffer for the serial port

IO_BUFFER:     DS        8    ;Reserve a buffer for the I/O

DBIT Directive

The DBIT Directive is used to reserve bits within the BIT

segment.  It can only be used when BSEG is the active segment.

The location counter of the segment is advanced by the value of

the directive.  Care should be taken to ensure that the location

counter does not advance beyond the limit of the segment.

The format for the DBIT Directive is: optional label, followed by

one or more spaces or tabs, followed by DBIT, followed by one or

more spaces or tabs, followed by a number, arithmetic expression,

or previously defined symbol (no forward references allowed),

followed by an optional comment.

Below  is  an example of using the DBIT Directive:

BSEG                ;Select the bit segment

DBIT      16        ;Label is optional

IO_MAP:        DBIT      32        ;Reserve a bit buffer for I/O

DB Directive

The DB Directive is used to store byte constants in the Program

Memory Space.  It can only be used when CSEG is the active

segment.

The format for the DB Directive is: optional label, followed by

one or more spaces or tabs, followed by DB, followed by one or

more spaces or tabs,  followed by the byte constants that are

5-5

separated by commas, followed by an optional comment.

The byte constants can be numbers, arithmetic expressions, symbol

values or ASCII literals.  ASCII literals have to be delimited by

apostrophes ( ' ), but they can be strung together up to the

length of the line.

Below are examples of using the DB Directive.  If an optional

label is used, its value will point to the first byte constant

listed.

COPYRGHT_MSG:

DB        '(c) Copyright, 1984'    ;ASCII Literal

RUNTIME_CONSTANTS:

DB        127,13,54,0,99           ;Table of constants

DB        17,32,239,163,49         ;Label is optional

MIXED:    DB        2*8,'MPG',2*16,'abc'     ;Can mix literals & no.

DW Directive

The DW Directive is used to store word constants in the Program

Memory Space.  It can only be used when CSEG is the active

segment.

The format for the DW Directive is: optional label, followed by

one or more spaces or tabs, followed by DW, followed by one or

more spaces or tabs, followed by the word constants that are

separated by commas, followed by an optional comment.

The word constants can be numbers, arithmetic expressions, symbol

values or ASCII literals.  ASCII literals must be delimited by

apostrophes ( ' ), but unlike the DB Directive, only a maximum of

two ASCII characters can be strung together.  The first character

is placed in the high byte of the word and the second character

is placed in the low byte.  If only one character is enclosed by

the apostrophes, a zero will be placed in the high byte of the

word.

Below are examples of using the DW Directive.  If an optional

label is used, its value will point to the high byte of the first

word constant listed.

JUMP_TABLE:    DW   RESET,START,END     ;Table of addresses

DW   TEST,TRUE,FALSE     ;Optional label

RADIX:         DW   'H',1000H           ;1st byte contains 0

;2nd byte contains 48H (H)

;3rd byte contains 10H

;4th byte contains 0

5.5. Miscellaneous Directives

ORG Directive

5-6

The ORG Directive is used to specify a value for the currently

active segment's location counter.  It cannot be used to select

segments like the directives above.  It can only be used within a

segment when the location counter needs to be changed.  Care

should be taken to ensure that the location counter does not

advance beyond the limit of the selected segment.

The format of the ORG Directive is: zero or more spaces or tabs,

followed by ORG, followed by one or more spaces or tabs, followed

by a number, arithmetic expression, or previously defined symbol

(no forward references are allowed), followed by an optional

comment.

Below are examples of the ORG directive.

ORG  1000H                ;Location counter set at 4096 decimal

ORG  RESET                ;Previously defined symbol

ORG  BASE + MODULE_NO     ;Arithmetic expression

USING DIRECTIVE

The USING Directive is used to specify which of the four General

Purpose Register banks is used in the code that follows the

directive.  It allows the use of the predefined register symbols

AR0 thru AR7 instead of the register's direct addresses.  It

should be noted that the actual register bank switching must

still be done in the code.  This directive simplifies the direct

addressing of a specified register bank.

The format of the USING Directive is: zero or more spaces or

tabs, followed by USING, followed by one or more spaces or tabs,

followed by a number, arithmetic expression, or previously

defined symbol (no forward references are allowed), followed by

an optional comment.

The number, arithmetic expression, or previously defined symbol

must result in a number between 0 and 3 in order to specify one

of the four register banks in the 8051.

5-7

The following table maps the specified value in the USING

directive with the direct addresses of the predefined symbols.

Predefined |        USING Value                  |

Symbol  |    0         1         2         3  |

-----------------------------------------------

AR0        0         8        16        24

AR1        1         9        17        25

AR2        2        10        18        26

AR3        3        11        19        27

AR4        4        12        20        28

AR5        5        13        21        29

AR6        6        14        22        30

AR7        7        15        23        31

Below are examples of the USING Directive:

USING     0         ;Select addresses for Bank 0

USING     1+1+1     ;Arithmetic expressions

END Directive

The END Directive is used to signal the end of the source program

to the Cross Assembler.  Every source program must have one and

only one END Directive.  A missing END Directive, as well as text

beyond the occurrence of the END Directive are not allowed and

will be flagged as errors.

The format of the END Directive is: zero or more spaces or tabs,

followed by END, followed by an optional comment. All text must

appear in the source program before the occurrence of the END

Directive.

Below is an example of the END Directive:

END         ;This is the End

5.6. Conditional Assembly Directives

IF, ELSE and ENDIF Directive

The IF, ELSE and ENDIF directives are used to define conditional

assembly blocks.  A conditional assembly block begins with an IF

statement and must end with the ENDIF directive.  In between the

IF statement and ENDIF directive can be any number of assembly

language statements, including directives, controls,

instructions, the ELSE directive and nested IF-ENDIF conditional

assembly blocks.

The IF statement starts with the keyword IF, followed by one or

more spaces or tabs, followed by a number, arithmetic expression,

or previously defined symbol (no forward references are allowed),

followed by an optional comment.  The number, arithmetic

5-8

expression or symbol is evaluated and if found to be TRUE (non-

zero),  the assembly language statements are translated up to the

next ELSE or ENDIF directives.  If the IF statement was evaluated

FALSE (zero), the assembly language statements are considered

null up to the next ELSE or ENDIF directives.

If an optional ELSE appears in the conditional assembly block,

the assembly language statements following are handled oppositely

from the assembly language statements following the IF statement.

In other words, if the IF statement was evaluated TRUE, the

statements following it are translated, while the statements

following the ELSE will be handled as if they were null.  On the

other hand, if the IF statement was evaluated FALSE, only the

assembly language statements following the ELSE directive would

be translated.

IF-ELSE-ENDIF conditional assembly blocks can be nested up to 255

levels deep.  The following are some examples of conditional

assembly blocks.  This first conditional assembly block simply

checks the symbol DEBUG.  If DEBUG is non-zero, the MOV and CALL

instructions will be translated by the Cross Assembler.

IF (DEBUG)

MOV    A,#25

CALL   OUTPUT

ENDIF

The next example used the optional ELSE directive.  If

SMALL_MODEL is zero, only the statements following the ELSE

directive will be translated.

IF (SMALL_MODEL)

MOV  R0,#BUFFER

MOV  A,@R0

ELSE

MOV  R0,#EXT_BUFFER

MOVX A,@R0

ENDIF

The last example shows nested conditional assembly blocks.

Conditional assembly blocks can be nested up to 255 levels deep.

Every level of nesting must have balanced IF-ENDIF statements.

_

IF (VERSION > 10)                             \

CALL DOUBLE_PRECISION                     |

CALL UPDATE_STATUS      _                 |

IF (DEBUG)                  \                |

CALL DUMP_REGISTERS       >  Nested       |

ENDIF                      _/   Block        |

ELSE                                            >  Outer Block

CALL SINGLE_PRECISION                     |

CALL UPDATE_STATUS      _                 |

IF (DEBUG)                  \                |

CALL DUMP_REGISTERS       >   Nested      |

ENDIF                      _/    Block       |

ENDIF                                        _/

5-9

CHAPTER 6

8051 CROSS ASSEMBLER CONTROLS

6.1. Introduction

Assembler controls are used to control where the Cross Assembler

gets its input source file, where it stores the object file, how

it formats and where it outputs the listing.

All Assembler controls are prefaced with a dollar sign, ($).  No

spaces or tabs are allowed between the dollar sign and the body

of the control.  Also, only one control per line is permitted.

Comments are allowed on the same line as an Assembler control.

There are two types of controls, Primary controls and General

controls.  Primary controls can be invoked only  once per

assembly.  If an attempt is made to change a previously invoked

primary control, the attempt is ignored.  For example, if

$NOPRINT is put on line 1 of the source file and $PRINT is put on

line 2, the $PRINT control will be ignored and the listing will

not be output.  General controls can be invoked any number of

times in a source program.

There are two legal forms for each Assembler control, the full

form and the abbreviated form.  The two forms can be used inter-

changeable in the source program.

Below is a description of each Assembler control.  Assembler

controls with common functionality are grouped together.

6.2. Assembler Control Descriptions

$DATE(date)

Places the ASCII string enclosed by parenthesis in the date

field of the page header.  The ASCII string can be from 0 to 9

characters long.

CONTROL:   $DATE(date)

ABBREV:    $DA(date)

TYPE:      Primary

DEFAULT:   No date in page header

EXAMPLES:  $DATE(1-JUL-84)

$DA(7/22/84)

6-1

$DEBUG(file)

$NODEBUG

These controls determine whether or not a MetaLink Absolute

Object Module format file is created. The MetaLink Absolute

Object Module format file is used in conjunction with

MetaLink's MetaICE series of in-circuit-emulators.  Among

other advantages, it provides powerful symbolic debug

capability in the emulator debug environment.  $NODEBUG

specifies that a MetaLink Absolute Object Module file will not

be created. $DEBUG specifies that a MetaLink Absolute Object

Module file will be created.  The $DEBUG control allows any

legal file name to be specified as the MetaLink Absolute

Object Module filename.  If no filename is specified, a

default name is used.  The default name used for the file is

the source file name root with a .DBG extension.  If the

$DEBUG control is used, both a MetaLink Absolute Object Module

file and a standard Intel Hexadecimal format object file can

be generated at the same time.  Refer to the $OBJECT control

description later in this chapter for information on

controlling the Hexadecimal format object file output.

CONTROL:   $DEBUG(file)

$NODEBUG

ABBREV:    $DB(file)

$NODB

DEFAULT:   $NODEBUG

TYPE:      Primary

EXAMPLES:  $DB(A:NEWNAME.ICE)

$DEBUG

$NOOBJECT

$EJECT

Places a form feed (ASCII 0CH) in the listing output.  The

$NOPAGING control will override this control.

CONTROL:   $EJECT

ABBREV:    $EJ

DEFAULT:   No form feeds in listing output

TYPE:      General

EXAMPLES:  $EJECT

$EJ

$INCLUDE(file)

Inserts a file in source program as part of the input source

program.  The file field in this control can be any legal file

designator. No extension is assumed, so the whole file name

must be specified.  Any number of files can be included in a

source program. Includes can be nested up to 8 level deep.  It

is important to note that this control inserts files, it does

not chain or concatenate files.

CONTROL:   $INCLUDE(file)

ABBREV:    $IC(file)

DEFAULT:   No file included in source program

TYPE:      General

EXAMPLES:  $INCLUDE(B:COMMON.EQU

$IC(TABLES.ASM)            ;Uses default drive

6-2

$LIST

$NOLIST

These controls determine whether or not the source program

listing is output or not. $LIST will allow the source program

listing to be output.  $NOLIST stops the source program

listing from being output.  The $NOPRINT control overrides the

$LIST control.

CONTROL:   $LIST

$NOLIST

ABBREV:    $LI

$NOLI

DEFAULT:   $LIST

TYPE:      General

EXAMPLES:  $NOLIST                 ;This will cause the included

$INCLUDE(COMMON.TBL)    ;file not to be listed

$LI                     ;Listing continues

$MOD51

$MOD52

$MOD44

$MOD515

$MOD512

$MOD517

$MOD152

$MOD451

$MOD452

$MOD751

$MOD752

$MOD154

$MOD252

$MOD521

$MOD552

$MOD652

$MOD851

$NOMOD

Recognizes predefined special function register symbols in the

source program.  This saves the user from having to define all

the registers in the source program. Appendix B lists the

symbols that are defined by these controls.  $NOMOD disables

the recognizing function.  These controls access a files of

the same name that are included with the MetaLink 8051 CROSS

ASSEMBLER distribution diskette.  When a $MOD control is used

in a source program, it is important that the $MOD file be

available to the Cross Assembler.  The Cross Assembler first

looks for the $MOD file on the default drive, if it isn't

found there, the Cross Assembler looks for it on the A: drive.

The components supported by each switch are:

$MOD51:  8051, 8751, 8031, 80C51, 80C31, 87C51, 9761, 8053

$MOD52:  8052, 8032, 8752

$MOD44:  8044, 8344, 8744

$MOD515: 80515, 80535, 80C515, 80C535

$MOD512: 80512, 80532

$MOD517: 80C517, 80C537

6-3

$MOD152: 80C152, 83C152, 80C157

$MOD451: 80C451. 83C451, 87C451

$MOD452: 80C452, 83C452, 87C452

$MOD752: 83C752, 87C752

$MOD751: 83C751, 87C751

$MOD154: 83C514, 80C154, 85C154

$MOD252: 80C252, 83C252, 87C252, 80C51FA, 83C51FA, 87C51FA,

83C51FB, 87C51FB

$MOD521: 80C521, 80C321, 87C521, 80C541, 87C541

$MOD552: 80C552, 83C552, 87C552

$MOD652: 80C652, 83C652

$MOD851: 80C851, 83C851

CONTROL:  $MOD51

$MOD52

$MOD44

$MOD152

$MOD515

$MOD512

$MOD451

$MOD452

$MOD751

$MOD752

$MOD154

$MOD252

$MOD521

$MOD552

$MOD652

$MOD517

$MOD851

$NOMOD

ABBREV:

DEFAULT:  $NOMOD

TYPE:     Primary

EXAMPLES: $MOD51

$MOD52

$MOD44

$MOD515

$MOD512

$MOD152

$MOD451

$MOD452

$MOD751

$MOD752

$MOD154

$MOD252

$MOD521

$MOD552

$MOD652

$MOD517

$MOD851

$NOMOD

$OBJECT(file)

$NOOBJECT

6-4

These controls determine whether or not a standard Intel

Hexadecimal format object file is created.  $NOOBJECT

specifies that an object file will not be created. $OBJECT

specifies that an object file will be created.  If other than

the default name is to be used for the object file, the

$OBJECT control allows any legal file name to be specified as

the object filename.  The default name used for the object

file is the source file name root with a .HEX extension.

CONTROL:   $OBJECT(file)

$NOOBJECT

ABBREV:    $OJ(file)

$NOOJ

DEFAULT:   $OBJECT(source.HEX)

TYPE:      Primary

EXAMPLES:  $OJ(A:NEWNAME.OBJ)

$NOOBJECT

$PAGING

$NOPAGING

These controls specify whether or not the output listing will

be broken into pages or will be output as one continuous

listing.  When the $NOPAGING control is used, the $EJECT and

$PAGELENGTH controls are ignored.  With the $PAGING control, a

form feed and header line is inserted into the output listing

whenever an $EJECT control is met, or whenever the number of

lines output on the current page exceeds the value specified

by the $PAGELENGTH control.  The header line contains source

file name, title (if $TITLE control was used), date (if $DATE

control was used) and page number.

CONTROL:   $PAGING

$NOPAGING

ABBREV:    $PI

$NOPI

DEFAULT:   $PAGING

TYPE:      Primary

EXAMPLES:  $PAGING

$NOPI

$PAGELENGTH(n)

Sets the maximum number of lines, (n), on a page of the output

listing.  If the maximum is exceeded, a form feed and page

header is inserted in the output listing.  This control allows

the number of lines per page to be set anywhere between 10 and

255.  If the number of lines specified is less than 10,

pagelength will be set to 10.  If the number of lines

specified is greater than 255, pagelength will be set to 255.

The $NOPAGING control will override this control.

CONTROL:   $PAGELENGTH(n)

ABBREV:    $PL(n)

DEFAULT:   $PAGELENGTH(60)

TYPE:      Primary

EXAMPLES:  $PAGELENGTH(48)

6-5

$PL(58)

$PAGEWIDTH(n)

Sets the maximum number of characters, (n), on a line of the

output listing.  This control allows the number of characters

per line to be set anywhere between 72 and 132.  If the number

specified is less than 72, the pagewidth is set at 72.  If the

number specified is greater than 132, the pagewidth is set at

132.  If the pagewidth is specified between 72 and 100 and the

line being output exceeds the pagewidth specification, the

line is truncated at the specified pagewidth and a carriage

return/line feed pair is inserted in the listing.  If the

pagewidth is specified to be greater than 100 and the line

being output exceed the pagewidth specification, a carriage

return/line feed pair is inserted at the specified pagewidth

and the line will continue to be listed on the next line

beginning at column 80.

CONTROL:   $PAGEWIDTH(n)

ABBREV:    $PW(n)

DEFAULT    $PAGEWIDTH(72)

TYPE:      Primary

EXAMPLES:  $PAGEWIDTH(132)

$PW(80)

$PRINT(file)

$NOPRINT

These controls determine whether or not a listing file is

created.  $NOPRINT  specifies that a listing file will not be

created. $PRINT specifies that an listing file will be

created.  If other than the default name is to be used for the

listing file, the $PRINT control allows any legal file name to

be specified as the listing filename.  The default name used

for the listing file is the source file name root with a .LST

extension.

CONTROL:   $PRINT(file)

$NOPRINT

ABBREV:    $PR

$NOPR

DEFAULT:   $PRINT(source.LST)

TYPE:      Primary

EXAMPLES:  $PRINT(A:CONTROL.OUT)

$NOPR

$SYMBOLS

$NOSYMBOLS

Selects whether or not the symbol table is appended to the

listing output.  $SYMBOLS causes the symbol table to be sorted

alphabetically by symbol, formatted and output to the listing

file.  Along with the symbol name, its value and type are

output.  Values are output in hexadecimal.  Types include NUMB

6-6

(number), ADDR (address), REG (register symbol) and ACC

(accumulator symbol).  If a symbol was of type ADDR, it

segment is also output as either C (code), D (data) or X

(external).  Other information listed with the symbols is NOT

USED (symbol defined but never referenced), UNDEFINED (symbol

referenced but never defined) and REDEFINEABLE (symbol defined

using the SET directive).  The type and value listed for a

REDEFINABLE symbol is that of its last definition in the

source program. $NOSYMBOLS does not output the symbol table.

CONTROL:   $SYMBOLS

$NOSYMBOLS

ABBREV:    $SB

$NOSB

DEFAULT:   $SYMBOLS

TYPE:      Primary

EXAMPLES:  $SB

$NOSYMBOLS

$TITLE(string)

Places the ASCII string enclosed by the parenthesis in the

title field of the page header.  The ASCII string can be from

0 to 64 characters long.  If the string is greater than 64

characters or if the width of the page will not support such a

long title, the title will be truncated.  If parentheses are

part of the string, they must be balanced.

CONTROL:   $TITLE(string)

ABBREV:    $TT(string)

DEFAULT:   No title in page header

TYPE:      Primary

EXAMPLES:  $TITLE(SAMPLE PROGRAM V1.2)

$TT(METALINK (TM) CROSS ASSEMBLER)

6-7

CHAPTER 7

8051 CROSS ASSEMBLER MACRO PROCESSOR

7.1. Introduction

Macros are useful for code that is used repetitively throughout

the program.  It saves the programmer the time and tedium of

having to specify the code every time it is used.  The code is

written only once in the macro definition and it can be used

anywhere in the source program any number of times by simply

using the macro name.

Sometimes there is confusion between macros and subroutines.

Subroutines are common routines that are written once by the

programmer and then accessed by CALLing them.  Subroutines are

usually used for longer and more complex routines where the

call/return overhead can be tolerated.  Macros are commonly used

for simpler routines or where the speed of in-line code is

required.

7.2. Macro Definition

Before a macro can be used, it first must be defined.  The macro

definition specifies a template that is inserted into the source

program whenever the macro name is encountered.  Macro

definitions can not be nested, but once a macro is defined, it

can be used in other macro definitions.  Macros used this way can

be nested up to nine levels deep.

The macro definition has three parts to it: 1) the macro header

which specifies the macro name and its parameter list, 2) the

macro body which is the part that is actually inserted into the

source program, and 3) the macro terminator.

The macro header has the following form:

name    MACRO     <parameter list>

The name field contains a unique symbol that it used to identify

the macro.  Whenever that symbol is encountered in the source

program, the Cross Assembler will automatically insert the macro

body in the source program at that point.  The name must be a

unique symbol that follows all the rules of symbol formation as

outlined in Chapter 2.

The MACRO field of the macro header contains the keyword MACRO.

This is used to notify the Cross Assembler that this is the

beginning of a macro definition.

7-1

The <parameter list> field of the macro header lists anywhere

from zero to 16 parameters that are used in the macro body and

are defined at assembly time.  The symbols used in the parameter

list are only used by the Cross Assembler during the storing of

the macro definition.  As a result, while symbols used in the

parameter list must be unique symbols that follow all the the

rules of symbol formation as outlined in Chapter 2, they can be

reissued in the parameter list of another macro definition

without conflict.  Parameter list items are separated from one

another by a comma.  The following are examples of macro

definition headers:

MULT_BY_16     MACRO                              (no parameters)

DIRECT_ADD     MACRO        DESTINATION,SOURCE    (two parameters)

The macro body contains the template that will replace the macro

name in the source program.  The macro body can contain

instructions, directives, conditional assembly statements or

controls.  As a matter of fact, the macro body can contain any

legal Cross Assembler construct as defined in Chapters 2, 4, 5

and 6.

There are two macro definition terminators: ENDM and EXITM.

Every macro definition must have an ENDM at the end of its

definition to notify the Cross Assembler that the macro

definition is complete.  The EXITM terminator is an alternative

ending of the macro that is useful with conditional assembly

statements.  When a EXITM is encountered in a program, all

remaining statements (to the ENDM) are ignored.

The following is an example of a macro definition that multiplies

the Accumulator by 16:

MULT_BY_16     MACRO

RL   A         ;* 2

RL   A         ;* 4

RL   A         ;* 8

RL   A         ;* 16

ENDM

The following is an example of a macro that adds two numbers

together.  This could be used by the programmer to do direct

memory to memory adds of external variables (create a virtual

instruction).

DIRECT_ADDX    MACRO    DESTINATION,SOURCE      (two parameters)

MOV  R0,#SOURCE

MOVX A,@R0

MOV  R1,A

MOV  R0,#DESTINATION

MOVX A,@R0

ADD  A,R1

MOVX @R0,A

ENDM

7-2

A final  macro definition example shows the use of the EXITM

macro terminator.  If CMOS is non-zero, the MOV and only the MOV

instruction will be translated by the Cross Assembler.

IDLE      MACRO

IF (CMOS)

MOV  PCON,#IDL

EXITM

ENDIF

JMP  $

ENDM

7-3

7.3. Special Macro Operators

There are four special macro operators that are defined below:

%         when the PERCENT sign prefaces a symbol in the

parameter list, the symbol's value is passed to

the macro's body instead of the symbol itself.

!         when the EXCLAMATION POINT precedes a character,

that character is handled as a literal and is

passed to the macro body with the EXCLAMATION

POINT removed.  This is useful when it is

necessary to pass a delimiter to the macro body.

For example, in the following parameter list, the

second parameter passed to the macro body would be

a COMMA ( , ):

GENERATE_INST       75,!,,STK_VALUE

&         when the AMPERSAND is used in the macro body, the

symbols on both sides of it are concatenated

together and the AMPERSAND is removed.

;;        when double SEMI-COLONS are used in a macro

definition, the comment preceded by the double

SEMI_COLONS will not be saved and thus will not

appear in the listing whenever the macro is

invoked.  Using the double SEMI-COLONS lowers the

memory requirement in storing the macro

definitions and should be used whenever possible.

Examples of using the above special macro operators follow in the

"Using Macros" section.

7.4. Using Macros

This section section discusses several situations that arise

using macros and how to handle them.  In general the discussion

uses examples to get the point across.  First the macro

definition is listed, then the source line program that will

invoke the macro and finally how the macro was expanded by the

Cross Assembler.

7.4.1. NESTING MACROS

The following shows a macro nested to a depth of three.

Remember, definitions cannot be nested.  Macros must be defined

before they are used in other macro definitions.

;MACRO DEFINITIONS

GET_EXT_BYTE     MACRO     EXT_ADDR

MOV    R0,#EXT_ADDR

MOVX   A,@R0

ENDM

7-4

ADD_EXT_BYTES    MACRO     EXT_DEST,EXT_SRC

GET_EXT_BYTE     EXT_DEST

MOV    R1,A

GET_EXT_BYTE     EXT_SRC

ADD    A,R1

ENDM

ADD_DIRECT_BYTES      MACRO     DESTINATION,SOURCE

IF (SMALL_MODEL)

MOV    A,SOURCE

ADD    A,DESTINATION

MOV    DESTINATION

ELSE

ADD_EXT_BYTES    DESTINATION,SOURCE

MOVX   @R0,A

ENDIF

ENDM

;USAGE IN PROGRAM

ADD_DIRECT_BYTES         127,128

;TRANSLATED MACRO

30 +1 ADD_DIRECT_BYTES         127,128

31 +1      IF (SMALL_MODEL)

32 +1           MOV    A,128

33 +1           ADD    A,127

34 +1           MOV    127

35 +1      ELSE

36 +2          ADD_EXT_BYTES  127,128

37 +3          GET_EXT_BYTE   127

0100 787F             38 +3          MOV    R0,#127

0102 E2               39 +3          MOVX   A,@R0

0103 F9               40 +2          MOV    R1,A

41 +3          GET_EXT_BYTE   128

0104 7880             42 +3          MOV    R0,#128

0106 E2               43 +3          MOVX   A,@R0

0107 29               44 +2          ADD    A,R1

0108 F2               45 +1          MOVX   @R0,A

46 +1      ENDIF

48

Two things should be pointed out from the above example.  First,

the order of the parameter list is important.  You must maintain

the the order of parameters from the macro definition if the

Cross Assembler is to translate the macro correctly.

Secondly, in order to pass parameters to nested macros, simply

use the same parameter symbol in the parameter list of the

definition.  For example, the parameter DESTINATION was passed

properly to the nested macros ADD_EXT_BYTES and GET_EXT_BYTE.

This occurred because in the macro definition of

ADD_DIRECT_BYTES, the parameter DESTINATION was specified in the

7-5

parameter lists of both ADD_EXT_BYTES and GET_EXT_BYTE.

7.4.2. LABELS IN MACROS

You have two choices for specifying labels in a macro body.  A

label can either be passed to the body as a parameter or it can

be generated within the body.  The following example shows both

ways.

;MACRO DEFINITION

MULTIPLE_SHIFT      MACRO          LABEL,LABEL_SUFFIX,COUNTER,N

COUNTER    SET    COUNTER+1    ;INCREMENT SUFFIX  FOR  NEXT

USAGE

LABEL:         MOV    R0,#N

SHIFT&LABEL_SUFFIX:   RL     A

DJNZ   R0,SHIFT&LABEL_SUFFIX

ENDM

;USAGE IN PROGRAM

MULTIPLE_SHIFT   LOOP_SHIFT,%COUNT,COUNT,4

;TRANSLATED MACRO

15 +1 MULTIPLE_SHIFT   LOOP_SHIFT,%COUNT,COUNT,4

0006            16 +1      COUNT   SET    COUNT+1

17 +1

0100 7804         18 +1 LOOP_SHIFT:  MOV    R0,#4

0102 23           19 +1 SHIFT5:      RL     A

0103 D8FD         20 +1              DJNZ    R0,SHIFT5

22

Points to note in the above example: 1) the double semi-colon

caused the comment not to be listed in the translated macro; 2)

the percent sign caused the value of COUNT (in this case the

value 5) to be passed to the macro body instead of the symbol;

and 3) the ampersand allowed two symbols to be concatenated to

form the label SHIFT5.

7-6

CHAPTER 8

8051 CROSS ASSEMBLER ERROR CODES

8.1. Introduction

When the Cross Assembler encounters an error in the source

program, it will emit an error message in the listing file.  If

the $NOPRINT control has been invoked, the error message will be

output to the screen.

There are basically two types of errors that are encountered by

the Cross Assembler, translation errors and I/O errors.  I/O

errors are usually fatal errors.  However, whenever an error is

detected, the Cross Assembler makes every effort possible to

continue with the assembly.

If it is possible to recover from the error and continue

assembling, the Cross Assembler will report the error, use a

default condition and continue on its way. However, when a fatal

error is encountered, it is impossible for the Cross Assembler to

proceed.  In this case, the Cross Assembler reports the error and

then aborts the assembly process.

Fatal I/O error messages are displayed on the screen and are of

the form:

FATAL ERROR opening <filename>

where <filename> would be replaced with the file designator

initially entered or read from the source program.  The cause of

this error is usually obvious, typically a typographical error or

the wrong drive specification.

Another fatal I/O error message is:

FATAL ERROR writing to <type> file

where <type> would be replaced with either "listing" or "object".

The cause of this error is usually either a write protected disk

or a full disk.

Translation error reports contain at least three lines.  The

first line is the source line in which the error was detected,

the second line is a pointer to the character, symbol, expression

or line that caused the error.  The final line is the error

message itself.  There may be more than one error message,

depending on the number of errors in the source line.  An example

of a source line with two errors in it follows:

0100 2323             26    START:  MOV     AB,@35

8-1

****----------------------------------------^---^

****ERROR #20: Illegal operand

****ERROR #20: Illegal operand

The errors are pointed out by the up-arrows ( ^ ).  For every up-

arrow there will be an error message.  Errors are ordered left to

right, so the first error message corresponds to the left-most

up-arrow and so on.  The error message includes an error number

and an description of the error.  The error number can be used as

an index to the more detailed error explanations that follow in

this chapter.

After the Cross Assembler has completed its translation process,

it will print an assembly complete message:

ASSEMBLY COMPLETE, nn ERRORS FOUND

If it was an error free assembly, in place of the "nn" above the

word "NO" will be output.  However, if errors were encountered

during the assembly process, the "nn" will be replaced with the

number of errors that were found (up to a maximum of 50).  In this

case, an error summary will follow in the listing file with all

the errors that were reported during the assembly.  An error

summary looks like the following:

ERROR SUMMARY:

Line #26, ERROR #20: Illegal operand

Line #26, ERROR #20: Illegal operand

The same error message that occurred after the source line

appears again prefaced by the source line number to aid in

tracking down the error in the source listing.

8.2. Explanation of Error Messages

ERROR #1: Illegal character

This error occurs when the Cross Assembler encounters a

character that is not part of its legal character set.  The

Cross Assembler character set can be found in Appendix D.

ERROR #2: Undefined symbol

This error occurs when the Cross Assembler tries to use a

symbol that hasn't been defined.  The two most common reasons

for this error are typographical errors and forward

references.

ERROR #3: Duplicate symbol

This error occurs when a previously defined symbol or a

reserved symbol is attempted to be defined again.  Refer to

Appendix C for the reserved words.  Also inspect the symbol in

the symbol table listing.  If the symbol doesn't appear there,

you are using a reserved word.  If the symbol does appear, its

original definition will be listed.

ERROR #4: Illegal digit for radix

A digit was encountered that is not part of the legal digits

8-2

for the radix specified.  Chapter 2 lists the legal digits for

each radix available.  Often this error occurs because a

symbol was started with a number instead of a letter, question

mark, or underscore.

ERROR #5: Number too large

The number specified, or the returned value of the expression,

exceeds 16-bit precision.  The largest value allowed is

65,535.

ERROR #6: Missing END directive

The source program must end with one and only one END

directive.  The END is placed after all the assembly line

statements.

ERROR #7: Illegal opcode/directive after label

The symbol after a label is not an opcode nor a directive that

allows labels.  The only thing permitted on a line after a

label is an instruction, the DS, DB or DW directives, or a

comment.  If none of these are found, this error will be

reported.

ERROR #8: Illegal assembly line

The assembly line doesn't begin with a symbol, label,

instruction mnemonic, control, directive, comment or null

line.  No attempt is made to translate such a line.

ERROR #9: Text beyond END directive

The END directive must be the last line of the source program.

Any text beyond the END line will cause this error.  Any such

text is ignore.  Text here is defined as any printable ASCII

characters.

ERROR #10: Illegal or missing expression

A number, symbol or arithmetic expression was expected, but it

was either found to be missing or the Cross Assembler was

unable to evaluate it properly.

ERROR #11: Illegal or missing expression operator

An arithmetic operator was expected but it is either missing

or it is not one of the legal operators specified in Chapter

2.

ERROR #12: Unbalanced parentheses

In evaluating an expression, the parentheses in the expression

were found not to balance.

ERROR #13: Illegal or missing expression value

In evaluating an expression, the Cross Assembler expected to

find either a number or a symbol, but it was either missing or

illegal.

ERROR #14: Illegal literal expression

This error occurs when a null ASCII literal string  is found.

A null ASCII literal is nothing more than two apostrophes

together ( '' ) and is illegal.

8-3

ERROR #15: Expression stack overflow

The expression stack has a depth of 32 values.  The expression

being evaluated exceeds this depth.  This is a very rare

error.  However, if you ever get it, divide the expression

into two or more expressions using the EQU directive.

ERROR #16: Division by zero

The expression being evaluated includes an attempt to divide

by zero.

ERROR #17: Illegal bit designator

A bit designator address was specified in the source program

and it points to an illegal bit address.  A bit designator

contains a byte address, followed by a PERIOD, followed by the

bit index into the byte address (e.g., ACC.7) as discussed in

Chapter 2.  This error can occur for one of two reasons.

First, if the number or a symbol that is used to specify the

byte address part of the bit designator is not a legal bit

addressable address, ERROR #17 will occur.  Second, if the bit

index into the byte address exceeds the number 7, again ERROR

#17 will be output.

ERROR #18: Target address exceeds relative address range

A Program Counter relative jump instruction (e.g., SJMP, JZ,

JNC, etc.) was decoded with the target address of the jump

exceeding the maximum possible forward jump of 127 bytes or

the maximum possible backward jump of 128 bytes.

ERROR #20: Illegal operand

The operand specified is not a legal operand for the

instruction.  Review the legal operands allowed for the

instruction.

ERROR #21: Illegal indirect register

R0 and R1 are the only primary legal indirect register.  This

error occurs when the indirect addressing mode designator (@)

is not followed by either R0, R1 or symbols that were defined

to be equivalent to either R0 or R1.  This error can also

occur in the MOVC A,@A+DPTR, MOVC A,@A+PC, MOVX A,@DPTR, MOVX

@DPTR,A and the JMP @A+DPTR instructions if the operands after

the indirect addressing mode designator ( @ ) aren't specified

properly.

ERROR #22: Missing operand delimiter

A COMMA operand delimiter is missing from the operand fields

of the instruction.

ERROR #23: Illegal or missing directive

This error occurs when the Cross Assembler cannot find a legal

directive.  The most common cause of this error is due to

leaving the COLON off a label.  As a result, the following

opcode mnemonic is attempted to be decoded as a directive.

ERROR #24: Attempting to EQUate a previously SET symbol

Once a symbol is defined using the SET directive, it cannot be

8-4

later redefined using the EQU directive.

ERROR #25: Attempting to SET a previously EQUated symbol

Once a symbol is defined using the EQU directive, it cannot be

redefined.  If you want the symbol to be redefineable, use the

SET directive.

ERROR #26: Illegal SET/EQU expression

The expression following the SET or EQU directive is illegal.

This typically occurs when an attempt is made to define a

symbol to be equivalent to an implicit register other than A,

R0, R1, R2, R3, R4, R5, R6 or R7.

ERROR #27: Illegal expression with forward reference

This error occurs when an expression contains a symbol that

hasn't been defined yet.  Move the symbol definition earlier

in the source file.

ERROR #28: Address exceeds segment range

The address specified exceeds 255 and you are in the DSEG,

BSEG, or ISEG.

ERROR #29: Expecting an EOL or COMMENT

The Cross Assembler has completed processing a legal assembly

language line and expected the line to be terminated with

either a COMMENT or a carriage return/line         feed pair.

ERROR #30: Illegal directive with current active segment

The specified directive is not legal in the active segment.

This can happen by trying to use the DBIT directive in other

than the BSEG, or using the DS directive in the BSEG.

ERROR #31: Only two character string allowed

This error occurs using the DW directive.  The maximum ASCII

literal allowed in a DW specification is a two character

string.

ERROR #32: Byte definition exceeds 255

This error occurs using the DB directive.  The value specified

in the DB specification cannot fit into a byte.

ERROR #33: Premature end of string

An ASCII literal string was not terminated properly with an

apostrophe.

ERROR #34: Illegal register bank number

This error occurs when the number specified with the USING

directive exceed 3.  Legal register bank numbers are: 0, 1, 2,

3.

ERROR #35: Include file nesting exceeds 8

The maximum number of nested include files is eight. You will

get this error if you exceed this limit.

ERROR #36: Illegal or missing argument

This error occurs when the syntax of a Cross Assembler control

8-5

requires an argument and it was either incorrectly specified

or is missing all together.

ERROR #37: Illegal control statement

The Cross Assembler does not recognize the specified control.

The legal controls are detailed in Chapter 6.

ERROR #38: Unable to open file

The Cross Assembler is unable to open the file as specified.

This is a fatal error which will abort the assembly process.

ERROR #39: Illegal file specification

The file specification is not a legal file designator.  Refer

to your DOS manual for a description of legal file

designators.  This is a fatal error which will abort the

assembly process.

ERROR #40: Program synchronization error

This error occurs when the Cross Assembler is generating the

object hex file and finds that the code segment location

counter is not advancing properly.  There are two cases where

this can happen.  First, if the source program uses ORG

directives and they are not placed in ascending order. Second,

if a generic CALL or JMP is made to a forward reference that

is actually defined later in the program to be a backward

reference. For example, the following code sequence will cause

this error due to the second reason:

BACK_REF:      NOP

CALL      FORWARD_REF

FORWARD_REF    EQU       BACK_REF

During the first pass, the generic CALL will be replaced with

a 3-byte LCALL instruction.  During the second pass, the

generic CALL will be replaced with a 2-byte ACALL instruction.

To prevent this kind of problem, use the generic CALLs and

JMPs with labeled targets, not EQU or SET defined symbols.

ERROR #41: Insufficient memory

This error occurs when there isn't enough memory to hold all

the symbols that have been generated by the source program.

If you have 96 Kbytes or more of RAM this will be a very rare

error.  Only a massive source program or numerous large macros

could potentially cause this error.  However, if this error

does occur, your best bet is to either buy more memory or to

break up your program into smaller pieces and share common

symbols with a common $INCLUDE file.

ERROR #42: More errors detected, not listed

The internal error buffer can hold 50 errors.  If more than 50

errors occur, only the first 50 will be reported.

ERROR #43: ENDIF without IF

The terminator of a conditional assembly block (ENDIF) was

recognized without seeing a matching IF.

8-6

ERROR #44: Missing ENDIF

A conditional assembly block was begun with an IF statement,

but no matching ENDIF was detected.

ERROR #45: Illegal or missing macro name

The MACRO keyword was recognized, but the symbol that is

supposed to precede the MACRO keyword was missing, an illegal

symbol or a duplicate symbol.

ERROR #46: Macro nesting too deep

Macros can be nested to a depth of 9 levels.  Exceeding this

limit will cause this error.

ERROR #47: Number of parameters doesn't match definition

In attempting to use a macro, the number of parameters in the

parameter list does not equal the number of parameters

specified in the macro definition.  They must match.

ERROR #48: Illegal parameter specification

This error typically occurs when a previously defined symbol

is used in the parameter list of the macro definition.

ERROR #49: Too many parameters

The maximum number of parameters in a macro parameter list is

sixteen.  This error occurs when you exceed that limit.

ERROR #50: Line exceeds 255 characters

The maximum length of a source line is 255 characters.  If a

carriage return/line feed pair is not detected in the first

256 characters of a line, this error is reported and the line

is truncated at 255 characters.

8-7

APPENDIX A

SAMPLE PROGRAM AND LISTING

A.1. Source File

;

;  8-bit by 8-bit signed multiply--byte signed multiply

;

;    This routine takes the signed byte in multiplicand and

;    multiplies it by the signed byte in multiplier and places

;    the signed 16-bit product in product_high and product_low.

;

;    This routine assumes 2s complement representation of signed

;    numbers.  The maximum numbers possible are then -128 and

;    +127.  Multiplying the possible maximum numbers together

;    easily fits into a 16-bit product, so no overflow test is

;    done on the answer.

;

;    Registers altered by routine: A, B, PSW.

;

;

;  Primary controls

$MOD51

$TITLE(BYTE SIGNED MULTIPLY)

$DATE(JUL-30-84)

$PAGEWIDTH(132)

$OBJECT(B:BMULB.OBJ)

;

;

;  Variable declarations

;

sign_flag  BIT     0F0H            ;sign of product

multiplier DATA    030H            ;8-bit multiplier

multiplicand       DATA    031H    ;8-bit multiplicand

product_high       DATA    032H    ;high byte of 16-bit answer

product_low        DATA    033H    ;low byte of answer

;

;

;

ORG     100H                    ;arbitrary start

;

byte_signed_multiply:

CLR     sign_flag       ;reset sign

MOV     A,multiplier    ;put multiplier in accumulator

JNB     ACC.7,positive  ;test sign bit of multiplier

CPL     A               ;negative--complement and

INC     A               ;add 1 to convert to positive

SETB    sign_flag       ;and set sign flag

;

A-1

positive:  MOV     B,multiplicand  ;put multiplicand in B register

JNB     B.7,multiply    ;test sign bit of multiplicand

XRL     B,#0FFh         ;negative--complement and

INC     B               ;add 1 to convert to positive

CPL     sign_flag       ;complement sign flag

;

multiply:  MUL     AB              ;do unsigned multiplication

;

sign_test: JNB     sign_flag,byte_signed_exit   ;if positive,done

XRL     B,#0FFh         ;else have to complement both

CPL     A               ;bytes of the product and inc

ADD     A,#1            ;add here because inc doesn't

JNC     byte_signed_exit      ;set the carry flag

INC     B               ;if add overflowed A, inc the

;high byte

byte_signed_exit:

MOV     product_high,B  ;save the answer

MOV     product_low,A

;

RET                     ;and return

END

A-2

A.2. Source File Listing

A-3

A-4

A-5

APPENDIX B

PRE-DEFINED BYTE AND BIT ADDRESSES

The following tables detail the pre-defined byte and bit addresses

for the 8051/8031 microcontrollers supported by the MetaLink family

of emulators.  Proliferation parts are delimited from the standard

MCS-51 definitions by asterisk ("*") boxes.

This list covers these microcontrollers:

8044  8031   8032   8051   8052   8053  80C154  80C321

8344  80C31  80C32  8751   8752   8753  83C154  80C521

8744                80C51  80C52        85C154  87C521

87C51

80C321  80C51FA(80C252)  80C452  80C152JA/JB/JC/JD    80C851

80C541  83C51FA(83C252)  83C452  83C152JA/JC          83C851

87C541  87C51FA(87C252)  87C452

80C451  80C652  80C552  83C751  83C752  80512  80515  80C515  80C517

83C451  83C652  83C552  87C751  87C752  80532  80535  80C535  80C537

87C451  87C652  87C552

B.1. Pre-defined Byte Addresses

P0        DATA      080H      ;PORT 0

SP        DATA      081H      ;STACK POINTER

DPL       DATA      082H      ;DATA POINTER - LOW BYTE

DPH       DATA      083H      ;DATA POINTER - HIGH BYTE

************************************************************************

for the 80C321/80C521

DPL1      DATA      084H      ;DATA POINTER LOW 1

DPH1      DATA      085H      ;DATA POINTER HIGH 1

DPS       DATA      086H      ;DATA POINTER SELECTION

************************************************************************

************************************************************************

for the 83C152/80C152

GMOD      DATA      084H      ;GSC MODE

TFIFO     DATA      085H      ;GSC TRANSMIT BUFFER

************************************************************************

************************************************************************

for the 80C517/80C537

WDTREL    DATA      086H      ;WATCHDOG TIMER RELOAD REG

************************************************************************

PCON      DATA      087H      ;POWER CONTROL

TCON      DATA      088H      ;TIMER CONTROL

TMOD      DATA      089H      ;TIMER MODE

TL0       DATA      08AH      ;TIMER 0 - LOW BYTE

TL1       DATA      08BH      ;TIMER 1 - LOW BYTE

B-1

************************************************************************

for the 83C751/83C752

RTL       DATA      08BH      ;TIMER 0 - LOW BYTE RELOAD

************************************************************************

TH0       DATA      08CH      ;TIMER 0 - HIGH BYTE

TH1       DATA      08DH      ;TIMER 1 - HIGH BYTE

************************************************************************

for the 83C751/83C752

RTH       DATA      08DH      ;TIMER 0 - HIGH BYTE RELOAD

************************************************************************

************************************************************************

for the 83C752

PWM       DATA      08EH      ;PULSE WIDTH MODULATION

************************************************************************

P1        DATA      090H      ;PORT 1

************************************************************************

for the 83C152/80C152

P5        DATA      091H      ;PORT 5

DCON0     DATA      092H      ;DMA CONTROL 0

DCON1     DATA      093H      ;DMA CONTROL 1

BAUD      DATA      094H      ;GSC BAUD RATE

ADR0      DATA      095H      ;GSC MATCH ADDRESS 0

************************************************************************

************************************************************************

for the 80C452/83C452

DCON0     DATA      092H      ;DMA CONTROL 0

DCON1     DATA      093H      ;DMA CONTROL 1

************************************************************************

************************************************************************

for the 80C517/80C537

DPSEL     DATA      092H      ;DATA POINTER SELECT REGISTER

************************************************************************

SCON      DATA      098H      ;SERIAL PORT CONTROL

SBUF      DATA      099H      ;SERIAL PORT BUFFER

************************************************************************

for the 83C751/83C752

I2CON     DATA      098H      ;I2C CONTROL

I2DAT     DATA      099H      ;I2C DATA

************************************************************************

************************************************************************

for the 80C517/80C537

IEN2      DATA      09AH      ;INTERRUPT ENABLE REGISTER 2

S1CON     DATA      09BH      ;SERIAL PORT CONTROL 1

S1BUF     DATA      09CH      ;SERIAL PORT BUFFER 1

S1REL     DATA      09DH      ;SERIAL RELOAD REG 1

************************************************************************

P2        DATA      0A0H      ;PORT 2

IE        DATA      0A8H      ;INTERRUPT ENABLE

B-2

************************************************************************

for the 80C51FA/83C51FA(83C252/80C252)

SADDR     DATA      0A9H      ;SLAVE INDIVIDUAL ADDRESS

************************************************************************

************************************************************************

for the 80515/80535 and 80C517/80C537

IP0       DATA      0A9H      ;INTERRUPT PRIORITY REGISTER 0

************************************************************************

************************************************************************

for the 80C321/80C521

WDS       DATA      0A9H      ;WATCHDOG SELECTION

WDK       DATA      0AAH      ;WATCHDOG KEY

************************************************************************

************************************************************************

for the 83C152/80C152

P6        DATA      0A1H      ;PORT 6

SARL0     DATA      0A2H      ;DMA SOURCE ADDR. 0 (LOW)

SARH0     DATA      0A3H      ;DMA SOURCE ADDR. 0 (HIGH)

IFS       DATA      0A4H      ;GSC INTERFRAME SPACING

ADR1      DATA      0A5H      ;GSC MATCH ADDRESS 1

************************************************************************

************************************************************************

for the 80C452/83C452

SARL0     DATA      0A2H      ;DMA SOURCE ADDR. 0 (LOW)

SARH0     DATA      0A3H      ;DMA SOURCE ADDR. 0 (HIGH)

************************************************************************

************************************************************************

for the 80C552/83C552

CML0      DATA      0A9H      ;COMPARE 0 - LOW BYTE

CML1      DATA      0AAH      ;COMPARE 1 - LOW BYTE

CML2      DATA      0ABH      ;COMPARE 2 - LOW BYTE

CTL0      DATA      0ACH      ;CAPTURE 0 - LOW BYTE

CTL1      DATA      0ADH      ;CAPTURE 1 - LOW BYTE

CTL2      DATA      0AEH      ;CAPTURE 2 - LOW BYTE

CTL3      DATA      0AFH      ;CAPTURE 3 - LOW BYTE

************************************************************************

P3        DATA      0B0H      ;PORT 3

************************************************************************

for the 83C152/80C152

SARL1     DATA      0B2H      ;DMA SOURCE ADDR. 1 (LOW)

SARH1     DATA      0B3H      ;DMA SOURCE ADDR. 1 (HIGH)

SLOTTM    DATA      0B4H      ;GSC SLOT TIME

ADR2      DATA      0B5H      ;GSC MATCH ADDRESS 2

************************************************************************

************************************************************************

for the 80C452/83C452

SARL1     DATA      0B2H      ;DMA SOURCE ADDR. 1 (LOW)

SARH1     DATA      0B3H      ;DMA SOURCE ADDR. 1 (HIGH)

************************************************************************

IP        DATA      0B8H      ;INTERRUPT PRIORITY

************************************************************************

for the 80C51FA/83C51FA(83C252/80C252)

B-3

SADEN     DATA      0B9H      ;SLAVE ADDRESS ENABLE

************************************************************************

************************************************************************

for the 80515/80535 and 80C517/80C537

IP1       DATA      0B9H      ;INTERRUPT PRIORITY REGISTER 1

IRCON     DATA      0C0H      ;INTERRUPT REQUEST CONTROL

CCEN      DATA      0C1H      ;COMPARE/CAPTURE ENABLE

CCL1      DATA      0C2H      ;COMPARE/CAPTURE REGISTER 1  - LOW BYTE

CCH1      DATA      0C3H      ;COMPARE/CAPTURE REGISTER 1  - HIGH BYTE

CCL2      DATA      0C4H      ;COMPARE/CAPTURE REGISTER 2  - LOW BYTE

CCH2      DATA      0C5H      ;COMPARE/CAPTURE REGISTER 2  - HIGH BYTE

CCL3      DATA      0C6H      ;COMPARE/CAPTURE REGISTER 3  - LOW BYTE

CCH3      DATA      0C7H      ;COMPARE/CAPTURE REGISTER 3  - HIGH BYTE

T2CON     DATA      0C8H      ;TIMER 2 CONTROL

CRCL      DATA      0CAH      ;COMPARE/RELOAD/CAPTURE - LOW BYTE

CRCH      DATA      0CBH      ;COMPARE/RELOAD/CAPTURE - HIGH BYTE

TL2       DATA      0CCH      ;TIMER 2 - LOW BYTE

TH2       DATA      0CDH      ;TIMER 2 - HIGH BYTE

************************************************************************

************************************************************************

for the 80C517/80C537

CC4EN     DATA      0C9H      ;COMPARE/CAPTURE 4 ENABLE

CCL4      DATA      0CEH      ;COMPARE/CAPTURE REGISTER 4  - LOW BYTE

CCH4      DATA      0CFH      ;COMPARE/CAPTURE REGISTER 4  - HIGH BYTE

************************************************************************

************************************************************************

for the RUPI-44

STS       DATA      0C8H      ;SIU STATUS REGISTER

SMD       DATA      0C9H      ;SERIAL MODE

RCB       DATA      0CAH      ;RECEIVE CONTROL BYTE

RBL       DATA      0CBH      ;RECEIVE BUFFER LENGTH

RBS       DATA      0CCH      ;RECEIVE BUFFER START

RFL       DATA      0CDH      ;RECEIVE FIELD LENGTH

STAD      DATA      0CEH      ;STATION ADDRESS

DMA_CNT   DATA      0CFH      ;DMA COUNT

************************************************************************

************************************************************************

for the 8052/8032, 80C51FA/83C51FA(83C252/80C252), 80C154/83C154

T2CON     DATA      0C8H      ;TIMER 2 CONTROL

************************************************************************

************************************************************************

for the 80C51FA/83C51FA(83C252/80C252)

T2MOD     DATA      0C9H      ;TIMER 2 MODE CONTROL

************************************************************************

************************************************************************

for the 8052/8032, 80C51FA/83C51FA(83C252/80C252), 80C154/83C154

RCAP2L    DATA      0CAH      ;TIMER 2 CAPTURE REGISTER, LOW BYTE

RCAP2H    DATA      0CBH      ;TIMER 2 CAPTURE REGISTER, HIGH BYTE

TL2       DATA      0CCH      ;TIMER 2 - LOW BYTE

TH2       DATA      0CDH      ;TIMER 2 - HIGH BYTE

************************************************************************

B-4

************************************************************************

for the 83C152/80C152

P4        DATA      0C0H      ;PORT 4

DARL0     DATA      0C2H      ;DMA DESTINATION ADDR. 0 (LOW)

DARH0     DATA      0C3H      ;DMA DESTINATION ADDR. 0 (HIGH)

BKOFF     DATA      0C4H      ;GSC BACKOFF TIMER

ADR3      DATA      0C5H      ;GSC MATCH ADDRESS 3

IEN1      DATA      0C8H      ;INTERRUPT ENABLE REGISTER 1

************************************************************************

************************************************************************

for the 80C452/83C452

P4        DATA      0C0H      ;PORT 4

DARL0     DATA      0C2H      ;DMA DESTINATION ADDR. 0 (LOW)

DARH0     DATA      0C3H      ;DMA DESTINATION ADDR. 0 (HIGH)

************************************************************************

************************************************************************

for the 80C451/83C451

P4        DATA      0C0H      ;PORT 4

P5        DATA      0C8H      ;PORT 5

************************************************************************

************************************************************************

for the 80512/80532

IRCON     DATA      0C0H      ;INTERRUPT REQUEST CONTROL

************************************************************************

************************************************************************

for the 80C552/83C552

P4        DATA      0C0H      ;PORT 4

P5        DATA      0C4H      ;PORT 5

ADCON     DATA      0C5H      ;A/D CONVERTER CONTROL

ADCH      DATA      0C6H      ;A/D CONVERTER HIGH BYTE

TM2IR     DATA      0C8H      ;T2 INTERRUPT FLAGS

CMH0      DATA      0C9H      ;COMPARE 0 - HIGH BYTE

CMH1      DATA      0CAH      ;COMPARE 1 - HIGH BYTE

CMH2      DATA      0CBH      ;COMPARE 2 - HIGH BYTE

CTH0      DATA      0CCH      ;CAPTURE 0 - HIGH BYTE

CTH1      DATA      0CDH      ;CAPTURE 1 - HIGH BYTE

CTH2      DATA      0CEH      ;CAPTURE 2 - HIGH BYTE

CTH3      DATA      0CFH      ;CAPTURE 3 - HIGH BYTE

************************************************************************

PSW       DATA      0D0H      ;PROGRAM STATUS WORD

************************************************************************

for the RUPI-44

NSNR      DATA      0D8H      ;SEND COUNT/RECEIVE COUNT

SIUST     DATA      0D9H      ;SIU STATE COUNTER

TCB       DATA      0DAH      ;TRANSMIT CONTROL BYTE

TBL       DATA      0DBH      ;TRANSMIT BUFFER LENGTH

TBS       DATA      0DCH      ;TRANSMIT BUFFER START

FIFO0     DATA      0DDH      ;THREE BYTE FIFO

FIFO1     DATA      0DEH

FIFO2     DATA      0DFH

************************************************************************

B-5

************************************************************************

for the 80C51FA/83C51FA(83C252/80C252)

CCON      DATA      0D8H      ;CONTROL COUNTER

CMOD      DATA      0D9H      ;COUNTER MODE

CCAPM0    DATA      0DAH      ;COMPARE/CAPTURE MODE FOR PCA MODULE 0

CCAPM1    DATA      0DBH      ;COMPARE/CAPTURE MODE FOR PCA MODULE 1

CCAPM2    DATA      0DCH      ;COMPARE/CAPTURE MODE FOR PCA MODULE 2

CCAPM3    DATA      0DDH      ;COMPARE/CAPTURE MODE FOR PCA MODULE 3

CCAPM4    DATA      0DEH      ;COMPARE/CAPTURE MODE FOR PCA MODULE 4

************************************************************************

************************************************************************

for the 80515/80535

ADCON     DATA      0D8H      ;A/D CONVERTER CONTROL

ADDAT     DATA      0D9H      ;A/D CONVERTER DATA

DAPR      DATA      0DAH      ;D/A CONVERTER PROGRAM REGISTER

************************************************************************

************************************************************************

for the 83C152/80C152

DARL1     DATA      0D2H      ;DMA DESTINATION ADDR. 1 (LOW)

DARH1     DATA      0D3H      ;DMA DESTINATION ADDR. 1 (HIGH)

TCDCNT    DATA      0D4H      ;GSC TRANSMIT COLLISION COUNTER

AMSK0     DATA      0D5H      ;GSC ADDRESS MASK 0

TSTAT     DATA      0D8H      ;TRANSMIT STATUS (DMA & GSC)

************************************************************************

************************************************************************

for the 80C452/83C452

DARL1     DATA      0D2H      ;DMA DESTINATION ADDR. 1 (LOW)

DARH1     DATA      0D3H      ;DMA DESTINATION ADDR. 1 (HIGH)

************************************************************************

************************************************************************

for the 80C451/83C451

P6        DATA      0D8H      ;PORT 6

************************************************************************

************************************************************************

for the 80512/80532

ADCON     DATA      0D8H      ;A/D CONVERTER CONTROL

ADDAT     DATA      0D9H      ;A/D CONVERTER DATA

DAPR      DATA      0DAH      ;D/A CONVERTER PROGRAM REGISTER

P6        DATA      0DBH      ;PORT 6

************************************************************************

************************************************************************

for the 83C751/83C752

I2CFG     DATA      0D8H      ;I2C CONFIGURATION

************************************************************************

************************************************************************

for the 80C552/83C552 and 80C652/83C652

S1CON     DATA      0D8H      ;SERIAL 1 CONTROL

S1STA     DATA      0D9H      ;SERIAL 1 STATUS

S1DAT     DATA      0DAH      ;SERIAL 1 DATA

S1ADR     DATA      0DBH      ;SERIAL 1 SLAVE ADDRESS

************************************************************************

B-6

************************************************************************

for the 80C517/80C537

CML0      DATA      0D2H      ;COMPARE REGISTER 0  - LOW BYTE

CMH0      DATA      0D3H      ;COMPARE REGISTER 0  - HIGH BYTE

CML1      DATA      0D4H      ;COMPARE REGISTER 1  - LOW BYTE

CMH1      DATA      0D5H      ;COMPARE REGISTER 1  - HIGH BYTE

CML2      DATA      0D6H      ;COMPARE REGISTER 2  - LOW BYTE

CMH2      DATA      0D7H      ;COMPARE REGISTER 2  - HIGH BYTE

ADCON0    DATA      0D8H      ;A/D CONVERTER CONTROL 0

ADDAT     DATA      0D9H      ;A/D CONVERTER DATA

DAPR      DATA      0DAH      ;D/A CONVERTER PROGRAM REGISTER

P7        DATA      0DBH      ;PORT 7

ADCON1    DATA      0DCH      ;A/D CONVERTER CONTROL 1

P8        DATA      0DDH      ;PORT 8

CTRELL    DATA      0DEH      ;COM TIMER REL REG - LOW BYTE

CTRELH    DATA      0DFH      ;COM TIMER REL REG - HIGH BYTE

************************************************************************

ACC       DATA      0E0H      ;ACCUMULATOR

************************************************************************

for the 83C152/80C152

BCRL0     DATA      0E2H      ;DMA BYTE COUNT 0 (LOW)

BCRH0     DATA      0E3H      ;DMA BYTE COUNT 0 (HIGH)

PRBS      DATA      0E4H      ;GSC PSEUDO-RANDOM SEQUENCE

AMSK1     DATA      0E5H      ;GSC ADDRESS MASK 1

RSTAT     DATA      0E8H      ;RECEIVE STATUS (DMA & GSC)

************************************************************************

************************************************************************

for the 80C452/83C452

BCRL0     DATA      0E2H      ;DMA BYTE COUNT 0 (LOW)

BCRH0     DATA      0E3H      ;DMA BYTE COUNT 0 (HIGH)

HSTAT     DATA      0E6H      ;HOST STATUS

HCON      DATA      0E7H      ;HOST CONTROL

SLCON     DATA      0E8H      ;SLAVE CONTROL

SSTAT     DATA      0E9H      ;SLAVE STATUS

IWPR      DATA      0EAH      ;INPUT WRITE POINTER

IRPR      DATA      0EBH      ;INPUT READ POINTER

CBP       DATA      0ECH      ;CHANNEL BOUNDARY POINTER

FIN       DATA      0EEH      ;FIFO IN

CIN       DATA      0EFH      ;COMMAND IN

************************************************************************

************************************************************************

for the 80515/80535

P4        DATA      0E8H      ;PORT 4

************************************************************************

************************************************************************

for the 80C451/83C451

CSR       DATA      0E8H      ;CONTROL STATUS

************************************************************************

************************************************************************

for the 80512/80532

P4        DATA      0E8H      ;PORT 4

************************************************************************

B-7

************************************************************************

for the 80C552/83C552

IEN1      DATA      0E8H      ;INTERRUPT ENABLE REGISTER 1

TM2CON    DATA      0EAH      ;T2 COUNTER CONTROL

CTCON     DATA      0EBH      ;CAPTURE CONTROL

TML2      DATA      0ECH      ;TIMER 2 - LOW BYTE

TMH2      DATA      0EDH      ;TIMER 2 - HIGH BYTE

STE       DATA      0EEH      ;SET ENABLE

RTE       DATA      0EFH      ;RESET/TOGGLE ENABLE

************************************************************************

************************************************************************

for the 80C51FA/83C51FA(83C252/80C252)

CL        DATA      0E9H      ;CAPTURE BYTE LOW

CCAP0L    DATA      0EAH      ;COMPARE/CAPTURE 0 LOW BYTE

CCAP1L    DATA      0EBH      ;COMPARE/CAPTURE 1 LOW BYTE

CCAP2L    DATA      0ECH      ;COMPARE/CAPTURE 2 LOW BYTE

CCAP3L    DATA      0EDH      ;COMPARE/CAPTURE 3 LOW BYTE

CCAP4L    DATA      0EEH      ;COMPARE/CAPTURE 4 LOW BYTE

************************************************************************

************************************************************************

for the 80C517/80C537

CTCON     DATA      0E1H      ;COM TIMER CONTROL REG

CML3      DATA      0E2H      ;COMPARE REGISTER 3  - LOW BYTE

CMH3      DATA      0E3H      ;COMPARE REGISTER 3  - HIGH BYTE

CML4      DATA      0E4H      ;COMPARE REGISTER 4  - LOW BYTE

CMH4      DATA      0E5H      ;COMPARE REGISTER 4  - HIGH BYTE

CML5      DATA      0E6H      ;COMPARE REGISTER 5  - LOW BYTE

CMH5      DATA      0E7H      ;COMPARE REGISTER 5  - HIGH BYTE

P4        DATA      0E8H      ;PORT 4

MD0       DATA      0E9H      ;MUL/DIV REG 0

MD1       DATA      0EAH      ;MUL/DIV REG 1

MD2       DATA      0EBH      ;MUL/DIV REG 2

MD3       DATA      0ECH      ;MUL/DIV REG 3

MD4       DATA      0EDH      ;MUL/DIV REG 4

MD5       DATA      0EEH      ;MUL/DIV REG 5

ARCON     DATA      0EFH      ;ARITHMETIC CONTROL REG

************************************************************************

B         DATA      0F0H      ;MULTIPLICATION REGISTER

************************************************************************

for the 80C154/83C154

IOCON     DATA      0F8H      ;I/O CONTROL REGISTER

************************************************************************

************************************************************************

for the 83C152/80C152

BCRL1     DATA      0F2H      ;DMA BYTE COUNT 1 (LOW)

BCRH1     DATA      0F3H      ;DMA BYTE COUNT 1 (HIGH)

RFIFO     DATA      0F4H      ;GSC RECEIVE BUFFER

MYSLOT    DATA      0F5H      ;GSC SLOT ADDRESS

IPN1      DATA      0F8H      ;INTERRUPT PRIORITY REGISTER 1

************************************************************************

B-8

************************************************************************

for the 83C851/80C851

EADRL     DATA      0F2H      ;EEPROM Address Register - Low Byte

EADRH     DATA      0F3H      ;EEPROM Address Register - High Byte

EDAT      DATA      0F4H      ;EEPROM Data Register

ETIM      DATA      0F5H      ;EEPROM Timer Register

ECNTRL    DATA      0F6H      ;EEPROM Control Register

************************************************************************

************************************************************************

for the 80C452/83C452

BCRL1     DATA      0F2H      ;DMA BYTE COUNT 1 (LOW)

BCRH1     DATA      0F3H      ;DMA BYTE COUNT 1 (HIGH)

ITHR      DATA      0F6H      ;INPUT FIFO THRESHOLD

OTHR      DATA      0F7H      ;OUTPUT FIFO THRESHOLD

IEP       DATA      0F8H      ;INTERRUPT PRIORITY

MODE      DATA      0F9H      ;MODE

ORPR      DATA      0FAH      ;OUTPUT READ POINTER

OWPR      DATA      0FBH      ;OUTPUT WRITE POINTER

IMIN      DATA      0FCH      ;IMMEDIATE COMMAND IN

IMOUT     DATA      0FDH      ;IMMEDIATE COMMAND OUT

FOUT      DATA      0FEH      ;FIFO OUT

COUT      DATA      0FFH      ;COMMAND OUT

************************************************************************

************************************************************************

for the 80515/80535

P5        DATA      0F8H      ;PORT 5

************************************************************************

************************************************************************

for the 80512/80532

P5        DATA      0F8H      ;PORT 5

************************************************************************

************************************************************************

for the 83C751/83C752

I2STA     DATA      0F8H      ;I2C STATUS

************************************************************************

************************************************************************

for the 80C552/83C552

IP1       DATA      0F8H      ;INTERRUPT PRIORITY REGISTER 1

PWM0      DATA      0FCH      ;PULSE WIDTH REGISTER 0

PWM1      DATA      0FDH      ;PULSE WIDTH REGISTER 1

PWMP      DATA      0FEH      ;PRESCALER FREQUENCY CONTROL

T3        DATA      0FFH      ;T3 - WATCHDOG TIMER

************************************************************************

************************************************************************

for the 80C517/80C537

CMEN      DATA      0F6H      ;COMPARE ENABLE

CML6      DATA      0F2H      ;COMPARE REGISTER 6  - LOW BYTE

CMH6      DATA      0F3H      ;COMPARE REGISTER 6  - HIGH BYTE

CML7      DATA      0F4H      ;COMPARE REGISTER 7  - LOW BYTE

CMH7      DATA      0F5H      ;COMPARE REGISTER 7  - HIGH BYTE

CMSEL     DATA      0F7H      ;COMPARE INPUT REGISTER

P5        DATA      0F8H      ;PORT 5

P6        DATA      0FAH      ;PORT 6

************************************************************************

B-9

************************************************************************

for the 80C51FA/83C51FA(83C252/80C252)

CH        DATA      0F9H      ;CAPTURE HIGH BYTE

CCAP0H    DATA      0FAH      ;COMPARE/CAPTURE 0 HIGH BYTE

CCAP1H    DATA      0FBH      ;COMPARE/CAPTURE 1 HIGH BYTE

CCAP2H    DATA      0FCH      ;COMPARE/CAPTURE 2 HIGH BYTE

CCAP3H    DATA      0FDH      ;COMPARE/CAPTURE 3 HIGH BYTE

CCAP4H    DATA      0FEH      ;COMPARE/CAPTURE 4 HIGH BYTE

************************************************************************

************************************************************************

for the 83C752

PWENA     DATA      0FEH      ;PULSE WIDTH ENABLE

************************************************************************

B-10

B.2. Pre-defined Bit Addresses

************************************************************************

for the 83C751/83C752

SCL       BIT       080H      ;P0.0 - I2C SERIAL CLOCK

SDA       BIT       081H      ;P0.1 - I2C SERIAL DATA

************************************************************************

IT0       BIT       088H      ;TCON.0 - EXT. INTERRUPT 0 TYPE

IE0       BIT       089H      ;TCON.1 - EXT. INTERRUPT 0 EDGE FLAG

IT1       BIT       08AH      ;TCON.2 - EXT. INTERRUPT 1 TYPE

IE1       BIT       08BH      ;TCON.3 - EXT. INTERRUPT 1 EDGE FLAG

TR0       BIT       08CH      ;TCON.4 - TIMER 0 ON/OFF CONTROL

TF0       BIT       08DH      ;TCON.5 - TIMER 0 OVERFLOW FLAG

TR1       BIT       08EH      ;TCON.6 - TIMER 1 ON/OFF CONTROL

TF1       BIT       08FH      ;TCON.7 - TIMER 1 OVERFLOW FLAG

************************************************************************

for the 83C751/83C752

C/T       BIT       08EH      ;TCON.6 - COUNTER OR TIMER OPERATION

GATE      BIT       08FH      ;TCON.7 - GATE TIMER

************************************************************************

************************************************************************

for the 80515/80535

INT3      BIT       090H      ;P1.0 - EXT. INTERRUPT 3/CAPT & COMP 0

INT4      BIT       091H      ;P1.1 - EXT. INTERRUPT 4/CAPT & COMP 1

INT5      BIT       092H      ;P1.2 - EXT. INTERRUPT 5/CAPT & COMP 2

INT6      BIT       093H      ;P1.3 - EXT. INTERRUPT 6/CAPT & COMP 3

INT2      BIT       094H      ;P1.4 - EXT. INTERRUPT 2

T2EX      BIT       095H      ;P1.5 - TIMER 2 EXT. RELOAD TRIGGER INP

CLKOUT    BIT       096H      ;P1.6 - SYSTEM CLOCK OUTPUT

T2        BIT       097H      ;P1.7 - TIMER 2 INPUT

************************************************************************

************************************************************************

for the 83C152/80C152

GRXD      BIT       090H      ;P1.0 - GSC RECEIVER DATA INPUT

GTXD      BIT       091H      ;P1.1 - GSC TRANSMITTER DATA OUTPUT

DEN       BIT       092H      ;P1.2 - DRIVE ENABLE TO ENABLE EXT DRIVE

TXC       BIT       093H      ;P1.3 - GSC EXTERNAL TRANSMIT CLOCK INPU

RXC       BIT       094H      ;P1.4 - GSC EXTERNAL RECEIVER CLOCK INPU

************************************************************************

************************************************************************

for the 83C552/80C552

CT0I      BIT       090H      ;P1.0 - CAPTURE/TIMER INPUT 0

CT1I      BIT       091H      ;P1.1 - CAPTURE/TIMER INPUT 1

CT2I      BIT       092H      ;P1.2 - CAPTURE/TIMER INPUT 2

CT3I      BIT       093H      ;P1.3 - CAPTURE/TIMER INPUT 3

T2        BIT       094H      ;P1.4 - T2 EVENT INPUT

RT2       BIT       095H      ;P1.5 - T2 TIMER RESET SIGNAL

SCL       BIT       096H      ;P1.6 - SERIAL PORT CLOCK LINE I2C

SDA       BIT       097H      ;P1.7 - SERIAL PORT DATA LINE I2C

************************************************************************

B-11

************************************************************************

for the 80C517/80C537

INT3      BIT       090H      ;P1.0 - EXT. INTERRUPT 3/CAPT & COMP 0

INT4      BIT       091H      ;P1.1 - EXT. INTERRUPT 4/CAPT & COMP 1

INT5      BIT       092H      ;P1.2 - EXT. INTERRUPT 5/CAPT & COMP 2

INT6      BIT       093H      ;P1.3 - EXT. INTERRUPT 6/CAPT & COMP 3

INT2      BIT       094H      ;P1.4 - EXT. INTERRUPT 2

T2EX      BIT       095H      ;P1.5 - TIMER 2 EXT. RELOAD TRIGGER INPU

CLKOUT    BIT       096H      ;P1.6 - SYSTEM CLOCK OUTPUT

T2        BIT       097H      ;P1.7 - TIMER 2 INPUT

************************************************************************

************************************************************************

for the 80C452/83C452 and 80C152/83C152

HLD       BIT       095H      ;P1.5 - DMA HOLD REQUEST I/O

HLDA      BIT       096H      ;P1.6 - DMA HOLD ACKNOWLEDGE OUTPUT

************************************************************************

************************************************************************

for the 83C751/83C752

INT0      BIT       095H      ;P1.5 - EXTERNAL INTERRUPT 0 INPUT

INT1      BIT       096H      ;P1.6 - EXTERNAL INTERRUPT 1 INPUT

T0        BIT       096H      ;P1.7 - TIMER 0 COUNT INPUT

************************************************************************

RI        BIT       098H      ;SCON.0 - RECEIVE INTERRUPT FLAG

TI        BIT       099H      ;SCON.1 - TRANSMIT INTERRUPT FLAG

RB8       BIT       09AH      ;SCON.2 - RECEIVE BIT 8

TB8       BIT       09BH      ;SCON.3 - TRANSMIT BIT 8

REN       BIT       09CH      ;SCON.4 - RECEIVE ENABLE

SM2       BIT       09DH      ;SCON.5 - SERIAL MODE CONTROL BIT 2

SM1       BIT       09EH      ;SCON.6 - SERIAL MODE CONTROL BIT 1

SM0       BIT       09FH      ;SCON.7 - SERIAL MODE CONTROL BIT 0

************************************************************************

for the 83C751/83C752

MASTER    BIT(READ) 099H      ;I2CON.1 - MASTER

STP       BIT(READ) 09AH      ;I2CON.2 - STOP

STR       BIT(READ) 09BH      ;I2CON.3 - START

ARL       BIT(READ) 09CH      ;I2CON.4 - ARBITRATION LOSS

DRDY      BIT(READ) 09DH      ;I2CON.5 - DATA READY

ATN       BIT(READ) 09EH      ;I2CON.6 - ATTENTION

RDAT      BIT(READ) 09FH      ;I2CON.7 - RECEIVE DATA

XSTP      BIT(WRITE)098H      ;I2CON.0 - TRANSMIT STOP

XSTR      BIT(WRITE)099H      ;I2CON.1 - TRANSMIT REPEATED START

CSTP      BIT(WRITE)09AH      ;I2CON.2 - CLEAR STOP

CSTR      BIT(WRITE)09BH      ;I2CON.3 - CLEAR START

CARL      BIT(WRITE)09CH      ;I2CON.4 - CLEAR ARBITRATION LOSS

CDR       BIT(WRITE)09DH      ;I2CON.5 - CLEAR DATA READY

IDLE      BIT(WRITE)09EH      ;I2CON.6 - GO IDLE

CXA       BIT(WRITE)09FH      ;I2CON.7 - CLEAR TRANSMIT ACTIVE

************************************************************************

EX0       BIT       0A8H      ;IE.0 - EXTERNAL INTERRUPT 0 ENABLE

ET0       BIT       0A9H      ;IE.1 - TIMER 0 INTERRUPT ENABLE

EX1       BIT       0AAH      ;IE.2 - EXTERNAL INTERRUPT 1 ENABLE

ET1       BIT       0ABH      ;IE.3 - TIMER 1 INTERRUPT ENABLE

ES        BIT       0ACH      ;IE.4 - SERIAL PORT INTERRUPT ENABLE

B-12

************************************************************************

for the 83C751/83C752

EI2       BIT       0ACH      ;IE.4 - SERIAL PORT INTERRUPT ENABLE

************************************************************************

************************************************************************

for the 8052/8032, 80C154/83C154, 80C252(80C51FA), 80515/80535

ET2       BIT       0ADH      ;TIMER 2 INTERRUPT ENABLE

************************************************************************

************************************************************************

for the 80C652/83C652

ES1       BIT       0ADH      ;IE.5 - SERIAL PORT 1 INTERRUPT ENABLE

************************************************************************

************************************************************************

for the 80C252(80C51FA)

EC        BIT       0AEH      ;IE.6 - ENABLE PCA INTERRUPT

************************************************************************

************************************************************************

for the 80515/80535

WDT       BIT       0AEH      ;IEN0.6 - WATCHDOG TIMER RESET

************************************************************************

************************************************************************

for the 83C552/80C552

ES1       BIT       0ADH      ;IEN0.5 - SERIAL PORT 1 INTERRUPT ENABLE

EAD       BIT       0AEH      ;IEN0.6 - ENABLE A/D INTERRUPT

************************************************************************

************************************************************************

for the 80C517/80C537

ET2       BIT       0ADH      ;IEN0.5 - TIMER 2 INTERRUPT ENABLE

WDT       BIT       0AEH      ;IEN0.6 - WATCHDOG TIMER RESET

************************************************************************

EA        BIT       0AFH      ;IE.7 - GLOBAL INTERRUPT ENABLE

RXD       BIT       0B0H      ;P3.0 - SERIAL PORT RECEIVE INPUT

TXD       BIT       0B1H      ;P3.1 - SERIAL PORT TRANSMIT OUTPUT

INT0      BIT       0B2H      ;P3.2 - EXTERNAL INTERRUPT 0 INPUT

INT1      BIT       0B3H      ;P3.3 - EXTERNAL INTERRUPT 1 INPUT

T0        BIT       0B4H      ;P3.4 - TIMER 0 COUNT INPUT

T1        BIT       0B5H      ;P3.5 - TIMER 1 COUNT INPUT

WR        BIT       0B6H      ;P3.6 - WRITE CONTROL FOR EXT. MEMORY

RD        BIT       0B7H      ;P3.7 - READ CONTROL FOR EXT. MEMORY

PX0       BIT       0B8H      ;IP.0 - EXTERNAL INTERRUPT 0 PRIORITY

PT0       BIT       0B9H      ;IP.1 - TIMER 0 PRIORITY

PX1       BIT       0BAH      ;IP.2 - EXTERNAL INTERRUPT 1 PRIORITY

PT1       BIT       0BBH      ;IP.3 - TIMER 1 PRIORITY

PS        BIT       0BCH      ;IP.4 - SERIAL PORT PRIORITY

************************************************************************

for the 80C154/83C154

PT2       BIT       0BCH      ;IP.5 - TIMER 2 PRIORITY

PCT       BIT       0BFH      ;IP.7 - INTERRUPT PRIORITY DISABLE

************************************************************************

B-13

************************************************************************

for the 80C652/83C652

PS1       BIT       0BDH      ;IP.5 - SERIAL PORT 1 PRIORITY

************************************************************************

************************************************************************

for the 80C51FA/83C51FA(83C252/80C252)

PT2       BIT       0BDH      ;IP.5 - TIMER 2 PRIORITY

PPC       BIT       0BEH      ;IP.6 - PCA PRIORITY

************************************************************************

************************************************************************

for the 80515/80535 and 80C517/80C537

EADC      BIT       0B8H      ;IEN1.0 - A/D CONVERTER INTERRUPT EN

EX2       BIT       0B9H      ;IEN1.1 - EXT. INTERRUPT 2 ENABLE

EX3       BIT       0BAH      ;IEN1.2 - EXT. INT 3/CAPT/COMP INT 0 EN

EX4       BIT       0BBH      ;IEN1.3 - EXT. INT 4/CAPT/COMP INT 1 EN

EX5       BIT       0BCH      ;IEN1.4 - EXT. INT 5/CAPT/COMP INT 2 EN

EX6       BIT       0BDH      ;IEN1.5 - EXT. INT 6/CAPT/COMP INT 3 EN

SWDT      BIT       0BEH      ;IEN1.6 - WATCHDOG TIMER START

EXEN2     BIT       0BFH      ;IEN1.7 - T2 EXT. RELOAD INTER START

IADC      BIT       0C0H      ;IRCON.0 - A/D CONVERTER INTER REQUEST

IEX2      BIT       0C1H      ;IRCON.1 - EXT. INTERRUPT 2 EDGE FLAG

IEX3      BIT       0C2H      ;IRCON.2 - EXT. INTERRUPT 3 EDGE FLAG

IEX4      BIT       0C3H      ;IRCON.3 - EXT. INTERRUPT 4 EDGE FLAG

IEX5      BIT       0C4H      ;IRCON.4 - EXT. INTERRUPT 5 EDGE FLAG

IEX6      BIT       0C5H      ;IRCON.5 - EXT. INTERRUPT 6 EDGE FLAG

TF2       BIT       0C6H      ;IRCON.6 - TIMER 2 OVERFLOW FLAG

EXF2      BIT       0C7H      ;IRCON.7 - TIMER 2 EXT. RELOAD FLAG

T2IO      BIT       0C8H      ;T2CON.0 - TIMER 2 INPUT SELECT BIT 0

T2I1      BIT       0C9H      ;T2CON.1 - TIMER 2 INPUT SELECT BIT 1

T2CM      BIT       0CAH      ;T2CON.2 - COMPARE MODE

T2R0      BIT       0CBH      ;T2CON.3 - TIMER 2 RELOAD MODE SEL BIT 0

T2R1      BIT       0CCH      ;T2CON.4 - TIMER 2 RELOAD MODE SEL BIT 1

I2FR      BIT       0CDH      ;T2CON.5 - EXT. INT 2 F/R EDGE FLAG

I3FR      BIT       0CEH      ;T2CON.6 - EXT. INT 3 F/R EDGE FLAG

T2PS      BIT       0CFH      ;T2CON.7 - PRESCALER SELECT BIT

************************************************************************

************************************************************************

for the 83C552/80C552

PS1       BIT       0BDH      ;IP0.5 - SIO1

PAD       BIT       0BEH      ;IP0.6 - A/D CONVERTER

CMSR0     BIT       0C0H      ;P4.0 - T2 COMPARE AND SET/RESET OUTPUTS

CMSR1     BIT       0C1H      ;P4.1 - T2 COMPARE AND SET/RESET OUTPUTS

CMSR2     BIT       0C2H      ;P4.2 - T2 COMPARE AND SET/RESET OUTPUTS

CMSR3     BIT       0C3H      ;P4.3 - T2 COMPARE AND SET/RESET OUTPUTS

CMSR4     BIT       0C4H      ;P4.4 - T2 COMPARE AND SET/RESET OUTPUTS

CMSR5     BIT       0C5H      ;P4.5 - T2 COMPARE AND SET/RESET OUTPUTS

CMT0      BIT       0C6H      ;P4.6 - T2 COMPARE AND TOGGLE OUTPUTS

CMT1      BIT       0C7H      ;P4.7 - T2 COMPARE AND TOGGLE OUTPUTS

CTI0      BIT       0C8H      ;TM2IR.0 - T2 CAPTURE 0

CTI1      BIT       0C9H      ;TM2IR.1 - T2 CAPTURE 1

CTI2      BIT       0CAH      ;TM2IR.2 - T2 CAPTURE 2

CTI3      BIT       0CBH      ;TM2IR.3 - T2 CAPTURE 3

CMI0      BIT       0CCH      ;TM2IR.4 - T2 COMPARATOR 0

CMI1      BIT       0CDH      ;TM2IR.5 - T2 COMPARATOR 1

CMI2      BIT       0CEH      ;TM2IR.6 - T2 COMPARATOR 2

T2OV      BIT       0CFH      ;TM2IR.7 - T2 OVERFLOW

B-14

************************************************************************

************************************************************************

for the RUPI-44

RBP       BIT       0C8H      ;STS.0 - RECEIVE BUFFER PROTECT

AM        BIT       0C9H      ;STS.1 - AUTO/ADDRESSED MODE SELECT

OPB       BIT       0CAH      ;STS.2 - OPTIONAL POLL BIT

BOV          BIT       0CBH      ;STS.3 - RECEIVE BUFFER OVERRUN

SI        BIT       0CCH      ;STS.4 - SIU INTERRUPT FLAG

RTS       BIT       0CDH      ;STS.5 - REQUEST TO SEND

RBE       BIT       0CEH      ;STS.6 - RECEIVE BUFFER EMPTY

TBF       BIT       0CFH      ;STS.7 - TRANSMIT BUFFER FULL

************************************************************************

************************************************************************

for the 8052/8032, 80C154/83C154, 80C51FA/83C51FA(83C252/80C252)

CAP2      BIT       0C8H      ;T2CON.0 - CAPTURE OR RELOAD SELECT

CNT2      BIT       0C9H      ;T2CON.1 - TIMER OR COUNTER SELECT

TR2       BIT       0CAH      ;T2CON.2 - TIMER 2 ON/OFF CONTROL

EXEN2     BIT       0CBH      ;T2CON.3 - TIMER 2 EXTERNAL ENABLE FLAG

TCLK      BIT       0CCH      ;T2CON.4 - TRANSMIT CLOCK SELECT

RCLK      BIT       0CDH      ;T2CON.5 - RECEIVE CLOCK SELECT

EXF2      BIT       0CEH      ;T2CON.6 - EXTERNAL TRANSITION FLAG

TF2       BIT       0CFH      ;T2CON.7 - TIMER 2 OVERFLOW FLAG

************************************************************************

************************************************************************

for the 83C152/80C152

EGSRV     BIT       0C8H      ;IEN1.0 - GSC RECEIVE VALID

EGSRE     BIT       0C9H      ;IEN1.1 - GSC RECEIVE ERROR

EDMA0     BIT       0CAH      ;IEN1.2 - DMA CHANNEL REQUEST 0

EGSTV     BIT       0CBH      ;IEN1.3 - GSC TRANSMIT VALID

EDMA1     BIT       0CCH      ;IEN1.4 - DMA CHANNEL REQUEST 1

EGSTE     BIT       0CDH      ;IEN1.5 - GSC TRANSMIT ERROR

************************************************************************

************************************************************************

for the 80512/80532

IADC      BIT       0C0H      ;IRCON.0 - A/D CONVERTER INTERRUPT REQ

************************************************************************

P         BIT       0D0H      ;PSW.0 - ACCUMULATOR PARITY FLAG

************************************************************************

for the 83C552/80C552

F1        BIT       0D1H      ;PSW.1 - FLAG 1

************************************************************************

************************************************************************

for the 80512/80532

F1        BIT       0D1H      ;PSW.1 - FLAG 1

MX0       BIT       0D8H      ;ADCON.0 - ANALOG INPUT CH SELECT BIT 0

MX1       BIT       0D9H      ;ADCON.1 - ANALOG INPUT CH SELECT BIT 1

MX2       BIT       0DAH      ;ADCON.2 - ANALOG INPUT CH SELECT BIT 2

ADM       BIT       0DBH      ;ADCON.3 - A/D CONVERSION MODE

BSY       BIT       0DCH      ;ADCON.4 - BUSY FLAG

BD        BIT       0DFH      ;ADCON.7 - BAUD RATE ENABLE

************************************************************************

OV        BIT       0D2H      ;PSW.2 - OVERFLOW FLAG

RS0       BIT       0D3H      ;PSW.3 - REGISTER BANK SELECT 0

B-15

RS1       BIT       0D4H      ;PSW.4 - REGISTER BANK SELECT 1

F0        BIT       0D5H      ;PSW.5 - FLAG 0

AC        BIT       0D6H      ;PSW.6 - AUXILIARY CARRY FLAG

CY        BIT       0D7H      ;PSW.7 - CARRY FLAG

************************************************************************

for the 80C51FA/83C51FA(83C252/80C252)

CCF0      BIT       0D8H      ;CCON.0 -PCA MODULE 0 INTERRUPT FLAG

CCF1      BIT       0D9H      ;CCON.1 -PCA MODULE 1 INTERRUPT FLAG

CCF2      BIT       0DAH      ;CCON.2 -PCA MODULE 2 INTERRUPT FLAG

CCF3      BIT       0DBH      ;CCON.3 -PCA MODULE 3 INTERRUPT FLAG

CCF4      BIT       0DCH      ;CCON.4 -PCA MODULE 4 INTERRUPT FLAG

CR        BIT       0DEH      ;CCON.6 - COUNTER RUN

CF        BIT       0DFH      ;PCA COUNTER OVERFLOW FLAG

************************************************************************

************************************************************************

for the RUPI-44

SER       BIT       0D8H      ;NSNR.0 - RECEIVE SEQUENCE ERROR

NR0       BIT       0D9H      ;NSNR.1 - RECEIVE SEQUENCE COUNTER-BIT 0

NR1       BIT       0DAH      ;NSNR.2 - RECEIVE SEQUENCE COUNTER-BIT 1

NR2       BIT       0DBH      ;NSNR.3 - RECEIVE SEQUENCE COUNTER-BIT 2

SES       BIT       0DCH      ;NSNR.4 - SEND SEQUENCE ERROR

NS0       BIT       0DDH      ;NSNR.5 - SEND SEQUENCE COUNTER-BIT 0

NS1       BIT       0DEH      ;NSNR.6 - SEND SEQUENCE COUNTER-BIT 1

NS2       BIT       0DFH      ;NSNR.7 - SEND SEQUENCE COUNTER-BIT 2

************************************************************************

************************************************************************

for the 80515/80535

MX0       BIT       0D8H      ;ADCON.0 - ANALOG INPUT CH SELECT BIT 0

MX1       BIT       0D9H      ;ADCON.1 - ANALOG INPUT CH SELECT BIT 1

MX2       BIT       0DAH      ;ADCON.2 - ANALOG INPUT CH SELECT BIT 2

ADM       BIT       0DBH      ;ADCON.3 - A/D CONVERSION MODE

BSY       BIT       0DCH      ;ADCON.4 - BUSY FLAG

CLK       BIT       0DEH      ;ADCON.5 - SYSTEM CLOCK ENABLE

BD        BIT       0DFH      ;ADCON.7 - BAUD RATE ENABLE

************************************************************************

************************************************************************

for the 80C652/83C652

CR0       BIT       0D8H      ;S1CON.0 - CLOCK RATE 0

CR1       BIT       0D9H      ;S1CON.1 - CLOCK RATE 1

AA        BIT       0DAH      ;S1CON.2 - ASSERT ACKNOWLEDGE

SI        BIT       0DBH      ;S1CON.3 - SIO1 INTERRUPT BIT

STO       BIT       0DCH      ;S1CON.4 - STOP FLAG

STA       BIT       0DDH      ;S1CON.5 - START FLAG

ENS1      BIT       0DEH      ;S1CON.6 - ENABLE SIO1

************************************************************************

B-16

************************************************************************

for the 83C152/80C152

DMA       BIT       0D8H      ;TSTAT.0 - DMA SELECT

TEN       BIT       0D9H      ;TSTAT.1 - TRANSMIT ENABLE

TFNF      BIT       0DAH      ;TSTAT.2 - TRANSMIT FIFO NOT FULL

TDN       BIT       0DBH      ;TSTAT.3 - TRANSMIT DONE

TCDT      BIT       0DCH      ;TSTAT.4 - TRANSMIT COLLISION DETECT

UR        BIT       0DDH      ;TSTAT.5 - UNDERRUN

NOACK     BIT       0DEH      ;TSTAT.6 - NO ACKNOWLEDGE

LNI       BIT       0DFH      ;TSTAT.7 - LINE IDLE

HBAEN     BIT       0E8H      ;RSTAT.0 - HARDWARE BASED ACKNOWLEDGE EN

GREN      BIT       0E9H      ;RSTAT.1 - RECEIVER ENABLE

RFNE      BIT       0EAH      ;RSTAT.2 - RECEIVER FIFO NOT EMPTY

RDN       BIT       0EBH      ;RSTAT.3 - RECEIVER DONE

CRCE      BIT       0ECH      ;RSTAT.4 - CRC ERROR

AE        BIT       0EDH      ;RSTAT.5 - ALIGNMENT ERROR

RCABT     BIT       0EEH      ;RSTAT.6 - RCVR COLLISION/ABORT DETECT

OR        BIT       0EFH      ;RSTAT.7 - OVERRUN

PGSRV     BIT       0F8H      ;IPN1.0 - GSC RECEIVE VALID

PGSRE     BIT       0F9H      ;IPN1.1 - GSC RECEIVE ERROR

PDMA0     BIT       0FAH      ;IPN1.2 - DMA CHANNEL REQUEST 0

PGSTV     BIT       0FBH      ;IPN1.3 - GSC TRANSMIT VALID

PDMA1     BIT       0FCH      ;IPN1.4 - DMA CHANNEL REQUEST 1

PGSTE     BIT       0FDH      ;IPN1.5 - GSC TRANSMIT ERROR

************************************************************************

************************************************************************

for the 80C452/83C452

OFRS      BIT       0E8H      ;SLCON.0 - OUTPUT FIFO CH REQ SERVICE

IFRS      BIT       0E9H      ;SLCON.1 - INPUT FIFO CH REQ SERVICE

FRZ       BIT       0EBH      ;SLCON.3 - ENABLE FIFO DMA FREEZE MODE

ICOI      BIT       0ECH      ;SLCON.4 - GEN INT WHEN IMMEDIATE COMMAN

OUT REGISTER IS AVAILABLE

ICII      BIT       0EDH      ;SLCON.5 - GEN INT WHEN A COMMAND IS

WRITTEN TO IMMEDIATE COMMAND IN REG

OFI       BIT       0EEH      ;SLCON.6 - ENABLE OUTPUT FIFO INTERRUPT

IFI       BIT       0EFH      ;SLCON.7 - ENABLE INPUT FIFO INTERRUPT

EFIFO     BIT       0F8H      ;IEP.0 - FIFO SLAVE BUS I/F INT EN

PDMA1     BIT       0F9H      ;IEP.1 - DMA CHANNEL REQUEST 1

PDMA0     BIT       0FAH      ;IEP.2 - DMA CHANNEL REQUEST 0

EDMA1     BIT       0FBH      ;IEP.3 - DMA CHANNEL 1 INTERRUPT ENABLE

EDMA0     BIT       0FCH      ;IEP.4 - DMA CHANNEL 0 INTERRUPT ENABLE

PFIFO     BIT       0FDH      ;IEP.5 - FIFO SLAVE BUS I/F INT PRIORITY

************************************************************************

************************************************************************

for the 80C451/83C451

IBF       BIT       0E8H      ;CSR.0 - INPUT BUFFER FULL

OBF       BIT       0E9H      ;CSR.1 - OUTPUT BUFFER FULL

IDSM      BIT       0EAH      ;CSR.2 - INPUT DATA STROBE

OBFC      BIT       0EBH      ;CSR.3 - OUTPUT BUFFER FLAG CLEAR

MA0       BIT       0ECH      ;CSR.4 - AFLAG MODE SELECT

MA1       BIT       0EDH      ;CSR.5 - AFLAG MODE SELECT

MB0       BIT       0EEH      ;CSR.6 - BFLAG MODE SELECT

MB1       BIT       0EFH      ;CSR.7 - BFLAG MODE SELECT

************************************************************************

************************************************************************

B-17

for the 83C751/83C752

CTO       BIT(READ) 0D8H      ;I2CFG.0 - CLOCK TIMING 0

CT1       BIT(READ) 0D9H      ;I2CFG.1 - CLOCK TIMING 1

T1RUN     BIT(READ) 0DCH      ;I2CFG.4 - START/STOP TIMER 1

MASTRQ    BIT(READ) 0DEH      ;I2CFG.6 - MASTER I2C

SLAVEN    BIT(READ) 0DFH      ;I2CFG.7 - SLAVE I2C

CT0       BIT(WRITE)0D8H      ;I2CFG.0 - CLOCK TIMING 0

CT1       BIT(WRITE)0D9H      ;I2CFG.1 - CLOCK TIMING 1

TIRUN     BIT(WRITE)0DCH      ;I2CFG.4 - START/STOP TIMER 1

CLRTI     BIT(WRITE)0DDH      ;I2CFG.5 - CLEAR TIMER 1 INTERRUPT FLAG

MASTRQ    BIT(WRITE)0DEH      ;I2CFG.6 - MASTER I2C

SLAVEN    BIT(WRITE)0DFH      ;I2CFG.7 - SLAVE I2C

RSTP      BIT(READ) 0F8H      ;I2STA.0 - XMIT STOP CONDITION

RSTR      BIT(READ) 0F9H      ;I2STA.1 - XMIT REPEAT STOP COND.

MAKSTP    BIT(READ) 0FAH      ;I2STA.2 - STOP CONDITION

MAKSTR    BIT(READ) 0FBH      ;I2STA.3 - START CONDITION

XACTV     BIT(READ) 0FCH      ;I2STA.4 - XMIT ACTIVE

XDATA     BIT(READ) 0FDH      ;I2STA.5 - CONTENT OF XMIT BUFFER

RIDLE     BIT(READ) 0FEH      ;I2STA.6 - SLAVE IDLE FLAG

************************************************************************

************************************************************************

for the 83C552/80C552

CR0       BIT       0D8H      ;S1CON.0 - CLOCK RATE 0

CR1       BIT       0D9H      ;S1CON.1 - CLOCK RATE 1

AA        BIT       0DAH      ;S1CON.2 - ASSERT ACKNOWLEDGE

SI        BIT       0DBH      ;S1CON.3 - SERIAL I/O INTERRUPT

STO       BIT       0DCH      ;S1CON.4 - STOP FLAG

STA       BIT       0DDH      ;S1CON.5 - START FLAG

ENS1      BIT       0DEH      ;S1CON.6 - ENABLE SERIAL I/O

ECT0      BIT       0E8H      ;IEN1.0 - ENABLE T2 CAPTURE 0

ECT1      BIT       0E9H      ;IEN1.1 - ENABLE T2 CAPTURE 1

ECT2      BIT       0EAH      ;IEN1.2 - ENABLE T2 CAPTURE 2

ECT3      BIT       0EBH      ;IEN1.3 - ENABLE T2 CAPTURE 3

ECM0      BIT       0ECH      ;IEN1.4 - ENABLE T2 COMPARATOR 0

ECM1      BIT       0EDH      ;IEN1.5 - ENABLE T2 COMPARATOR 1

ECM2      BIT       0EEH      ;IEN1.6 - ENABLE T2 COMPARATOR 2

ET2          BIT       0EFH      ;IEN1.7 - ENABLE T2 OVERFLOW

PCT0      BIT       0F8H      ;IP1.0 - T2 CAPTURE REGISTER 0

PCT1      BIT       0F9H      ;IP1.1 - T2 CAPTURE REGISTER 1

PCT2      BIT       0FAH      ;IP1.2 - T2 CAPTURE REGISTER 2

PCT3      BIT       0FBH      ;IP1.3 - T2 CAPTURE REGISTER 3

PCM0      BIT       0FCH      ;IP1.4 - T2 COMPARATOR 0

PCM1      BIT       0FDH      ;IP1.5 - T2 COMPARATOR 1

PCM2      BIT       0FEH      ;IP1.6 - T2 COMPARATOR 2

PT2          BIT       0FFH      ;IP1.7 - T2 OVERFLOW

************************************************************************

B-18

************************************************************************

for the 80C517/80C537

F1        BIT       0D1H      ;PSW.1 - FLAG 1

MX0       BIT       0D8H      ;ADCON0.0 - ANALOG INPUT CH SELECT BIT 0

MX1       BIT       0D9H      ;ADCON0.1 - ANALOG INPUT CH SELECT BIT 1

MX2       BIT       0DAH      ;ADCON0.2 - ANALOG INPUT CH SELECT BIT 2

ADM       BIT       0DBH      ;ADCON0.3 - A/D CONVERSION MODE

BSY       BIT       0DCH      ;ADCON0.4 - BUSY FLAG

CLK       BIT       0DEH      ;ADCON0.5 - SYSTEM CLOCK ENABLE

BD        BIT       0DFH      ;ADCON0.7 - BAUD RATE ENABLE

************************************************************************

************************************************************************

for the 80C154/83C154

ALF       BIT       0F8H      ;IOCON.0 - CPU POWER DOWN MODE CONTROL

P1F       BIT       0F9H      ;IOCON.1 - PORT 1 HIGH IMPEDANCE

P2F       BIT       0FAH      ;IOCON.2 - PORT 2 HIGH IMPEDANCE

P3F       BIT       0FBH      ;IOCON.3 - PORT 3 HIGH IMPEDANCE

IZC       BIT       0FCH      ;IOCON.4 - 10K TO 100 K OHM SWITCH (P1-3)

SERR      BIT       0FDH      ;IOCON.5 - SERIAL PORT RCV ERROR FLAG

T32       BIT       0FEH      ;IOCON.6 - 32 BIT TIMER SWITCH

WDT       BIT       0FFH      ;IOCON.7 - WATCHDOG TIMER CONTROL

*************************************************************************

B-19

APPENDIX C

RESERVED SYMBOLS

The following is a list of reserved symbols used by the Cross

Assembler.  These symbols cannot be redefined.

A               AB              ACALL           ADD

ADDC            AJMP            AND             ANL

AR0             AR1             AR2             AR3

AR4             AR5             AR6             AR7

BIT             BSEG            C               CALL

CJNE            CLR             CODE            CPL

CSEG            DA              DATA            DB

DBIT            DEC             DIV             DJNZ

DPTR            DS              DSEG            DW

END             EQ              EQU             GE

GT              HIGH            IDATA           INC

ISEG            JB              JBC             JC

JMP             JNB             JNC             JNZ

JZ              LCALL           LE              LJMP

LOW             LT              MOD             MOV

MOVC            MOVX            MUL             NE

NOP             NOT             OR              ORG

ORL             PC              POP             PUSH

R0              R1              R2              R3

R4              R5              R6              R7

RET             RETI            RL              RLC

RR              RRC             SET             SETB

SHL             SHR             SJMP            SUBB

SWAP            USING           XCH             XCHD

XDATA           XOR             XRL             XSEG

C-1

APPENDIX D

CROSS ASSEMBLER CHARACTER SET

---------------------------+----------------+-------------------

|   PRINTABLE   |    ASCII CODE

CHARACTER NAME      |     FORM      |   HEX  |  DECIMAL

---------------------------+----------------+---------+----------

Horizontal Tab           |               |   09   |     9

Line Feed                |               |   0A   |    10

Carriage Return          |               |   0D   |    13

Space                    |               |   20   |    32

Exclamation Point        |       !       |   21   |    33

Pound Sign               |       #       |   23   |    35

Dollar Sign              |       $       |   24   |    36

Percent Sign             |       %       |   25   |    37

Ampersand                |       &       |   26   |    38

Apostrophe               |       '       |   27   |    39

Left Parenthesis         |       (       |   28   |    40

Right Parenthesis        |       )       |   29   |    41

Asterisk                 |       *       |   2A   |    42

Plus sign                |       +       |   2B   |    43

Comma                    |       ,       |   2C   |    44

Hyphen                   |       -       |   2D   |    45

Period                   |       .       |   2E   |    46

Slash                    |       /       |   2F   |    47

Number 0                 |       0       |   30   |    48

"    1                 |       1       |   31   |    49

"    2                 |       2       |   32   |    50

"    3                 |       3       |   33   |    51

"    4                 |       4       |   34   |    52

"    5                 |       5       |   35   |    53

"    6                 |       6       |   36   |    54

"    7                 |       7       |   37   |    55

"    8                 |       8       |   38   |    56

"    9                 |       9       |   39   |    57

Colon                    |       :       |   3A   |    58

Semi-colon               |       ;       |   3B   |    59

Left Angle Bracket       |       <       |   3C   |    60

Equal Sign               |       =       |   3D   |    61

Right Angle Bracket      |       >       |   3E   |    62

Question Mark            |       ?       |   3F   |    63

At Sign                  |       @       |   40   |    64

Upper Case A             |       A       |   41   |    65

"    "   B             |       B       |   42   |    66

"    "   C             |       C       |   43   |    67

"    "   D             |       D       |   44   |    68

"    "   E             |       E       |   45   |    69

"    "   F             |       F       |   46   |    70

"    "   G             |       G       |   47   |    71

"    "   H             |       H       |   48   |    72

D-1

---------------------------+----------------+-------------------

|   PRINTABLE   |    ASCII CODE

CHARACTER NAME      |     FORM      |   HEX  |  DECIMAL

---------------------------+----------------+---------+----------

Upper Case I             |       I       |   49   |    73

"    "   J             |       J       |   4A   |    74

"    "   K             |       K       |   4B   |    75

"    "   L             |       L       |   4C   |    76

"    "   M             |       M       |   4D   |    77

"    "   N             |       N       |   4E   |    78

"    "   O             |       O       |   4F   |    79

"    "   P             |       P       |   50   |    80

"    "   Q             |       Q       |   51   |    81

"    "   R             |       R       |   52   |    82

"    "   S             |       S       |   53   |    83

"    "   T             |       T       |   54   |    84

"    "   U             |       U       |   55   |    85

"    "   V             |       V       |   56   |    86

"    "   W             |       W       |   57   |    87

"    "   X             |       X       |   58   |    88

"    "   Y             |       Y       |   59   |    89

"    "   Z             |       Z       |   5A   |    90

Underscore               |       _       |   5F   |    95

Lower Case A             |       a       |   61   |    97

"    "   B             |       b       |   62   |    98

"    "   C             |       c       |   63   |    99

"    "   D             |       d       |   64   |   100

"    "   E             |       e       |   65   |   101

"    "   F             |       f       |   66   |   102

"    "   G             |       g       |   67   |   103

"    "   H             |       h       |   68   |   104

"    "   I             |       i       |   69   |   105

"    "   J             |       j       |   6A   |   106

"    "   K             |       k       |   6B   |   107

"    "   L             |       l       |   6C   |   108

"    "   M             |       m       |   6D   |   109

"    "   N             |       n       |   6E   |   110

"    "   O             |       o       |   6F   |   111

"    "   P             |       p       |   70   |   112

"    "   Q             |       q       |   71   |   113

"    "   R             |       r       |   72   |   114

"    "   S             |       s       |   73   |   115

"    "   T             |       t       |   74   |   116

"    "   U             |       u       |   75   |   117

"    "   V             |       v       |   76   |   118

"    "   W             |       w       |   77   |   119

"    "   X             |       x       |   78   |   120

"    "   Y             |       y       |   79   |   121

"    "   Z             |       z       |   7A   |   122

D-2

INDEX

A

ASCII Literals, 2-6

Assembler

Comments, 2-6

Control Description ($), 6-1

Controls, 2-3

Directives, 2-3

Error codes/messages, 8-1

Labels, 2-2

Location Counter, 2-7

Numbers, 2-7

Operators, 2-7

Running it, 3-1

Symbols, 2-1

Syntax Summary, 2-7

B

Bit Addressing, 2-6

C

Character Set, D-1

Comments, 2-6

Control Description ($)

DATE, 6-1

DEBUG, 6-2

EJECT, 6-2

INCLUDE, 6-2

LIST, 6-3

MOD152, 6-3

MOD154, 6-3

MOD252, 6-3

MOD44, 6-3

MOD451, 6-3

MOD452, 6-3

MOD51, 6-3

MOD512, 6-3

MOD515, 6-3

MOD517, 6-3

MOD52, 6-3

MOD521, 6-3

MOD552, 6-3

MOD652, 6-3

MOD751, 6-3

MOD752, 6-3

MOD851, 6-3

NODEBUG, 6-2

NOLIST, 6-3

NOMOD, 6-3

NOOBJECT, 6-5

NOPAGING, 6-5

NOPRINT, 6-6

NOSYMBOLS, 6-6

OBJECT, 6-5

1

PAGELENGTH, 6-5

PAGEWIDTH, 6-6

PAGING, 6-5

PRINT, 6-6

SYMBOLS, 6-6

TITLE, 6-7

Controls

Description, 6-1

Introduction, 2-3, 6-1

D

Directive

Assembler, 2-3

Conditional Assembly, 5-8

Introduction, 5-1

Memory Reservation, 5-5

Miscellaneous, 5-7

Segment Selection, 5-4

Storage, 5-5

Symbol, 5-1

Directives

BIT, 5-2

BSEG, 5-4

CODE, 5-2

CSEG, 5-4

DATA, 5-3

DB, 5-5

DBIT, 5-5

DS, 5-5

DSEG, 5-4

DW, 5-6

ELSE, 5-8

END, 5-8

ENDIF, 5-8

EQU, 5-1

IDATA, 5-3

IF, 5-8

ISEG, 5-4

ORG, 5-7

SET, 5-1

XDATA, 5-3

XSEG, 5-4

E

Error Codes

Explainations, 8-2

Introduction, 8-1

Numerical Listing, 8-2

Executing

Assembler, 3-1

Assembler Example, 3-3

2

F

File

ASM51 Cross Assembler Diskette, 3-1

Object, 2-11

Source Listing, 2-10

Source Listing Sample, A-4

H

Hardware

Requirements, 3-1

I

Instruction

BYT, 4-3

CYC, 4-3

Dest/Source ADDR Mode, 4-2

HEX Opcode, 4-3

Language Form, 4-2

Mnemonics, 2-4, 4-1

Notation, 4-1

Operation, 4-1

PSW, 4-3

Set, 4-4

Introduction

Controls, 6-1

Cross Assembler, 2-1

Directives, 5-1

Error codes/messages, 8-1

Macro Processor, 7-1

L

Labels, 2-2

Location Counter, 2-7

M

Macro

Definition, 7-1

Special Operators, 7-4

Using, Labels, 7-6

Using, Nesting, 7-4

Using them, 7-4

Macro Processor

Introduction, 7-1

Microcontroller

Architecture MCS-51, 1-2

Background MCS-51, 1-1

Supported, 1-4

Mnemonics

MCS-51, 2-4

Notations, 4-1

Summary, 4-4

3

N

Numbers, 2-7

O

Operators, 2-7

Overview

Cross Assembler, 2-1

Symbols, 2-1

P

Program Sample, A-1

R

Reserved Symbols, 2-2

S

Sample program, A-1

Symbols

Directive Definition, 5-1

Labels, 2-2

Overview, 2-1

Reserved, 2-2

Reserved list, C-1

Syntax Summary, 2-7

System

DOS hints, 3-3

4